| Document Title | Requirements on DIO Driver |
| --- | --- |
| Document Owner | AUTOSAR |
| Document Responsibility | AUTOSAR |
| Document Identification No | 191 |

| Document Status | published |
| --- | --- |
| Part of AUTOSAR Standard | Classic Platform |
| Part of Standard Release | R23-11 |

| Document Change History |  |  |  |
| --- | --- | --- | --- |
| Date | Release | Changed by | Description |
| 2023-11-23 | R23-11 | AUTOSAR Release Management | - Editorial changes |
| 2022-11-24 | R22-11 | AUTOSAR Release Management | - No content changes |
| 2021-11-25 | R21-11 | AUTOSAR Release Management | - No content changes |
| 2020-11-30 | R20-11 | AUTOSAR Release Management | - No content changes |
| 2019-11-28 | R19-11 | AUTOSAR Release Management | - No content changes - Changed Document Status from Final to published |
| 2018-10-31 | 4.4.0 | AUTOSAR Release Management | - Editorial changes |
| 2017-12-08 | 4.3.1 | AUTOSAR Release Management | - Removed references to BMW and HIS documents - Editorial changes |
| 2016-11-30 | 4.3.0 | AUTOSAR Release Management | - Add section 5 Requirements Tracing |
| 2015-07-31 | 4.2.2 | AUTOSAR Release Management | - Editorial changes |
| 2014-10-31 | 4.2.1 | AUTOSAR Release Management | - Editorial changes |
| 2013-03-15 | 4.1.1 | AUTOSAR Administration | - Requirement IDs changed - Updated for requirements traceability |
| 2011-12-22 | 4.0.3 | AUTOSAR Administration | - Added [SRS_Dio_12900] |
| 2010-09-30 | 3.1.5 | AUTOSAR Administration | - Legal disclaimer revised |
| 2008-08-13 | 3.1.1 | AUTOSAR Administration | - Legal disclaimer revised |
| 2007-12-21 | 3.0.1 | AUTOSAR Administration | - Document meta information extended - Small layout adaptations made |
| 2007-01-24 | 2.1.15 | AUTOSAR Administration | - "Advice for users" revised - "Revision Information" added |
| 2006-11-28 | 2.1 | AUTOSAR Administration | - Legal disclaimer revised |
| 2006-05-16 | 2.0 | AUTOSAR Administration | - Release as a separate document. The SRS SPAL V1.0.0 has been split into 15 - independent documents for Release 2.0 |
| 2005-05-31 | 1.0 | AUTOSAR Administration | - Initial release as a part of the SRS SPAL - V1.0.0 |

Disclaimer

This work (specification and/or software implementation) and the material contained in it, as released by AUTOSAR, is for the purpose of information only. AUTOSAR and the companies that have contributed to it shall not be liable for any use of the work.

The material contained in this work is protected by copyright and other types of intel- lectual property rights. The commercial exploitation of the material contained in this work requires a license to such intellectual property rights.

This work may be utilized or reproduced without any modification, in any form or by any means, for informational purposes only. For any other purpose, no part of the work may be utilized or reproduced, in any form or by any means, without permission in writing from the publisher.

The work has been developed for automotive applications only. It has neither been developed, nor tested for non-automotive applications.

The word AUTOSAR and the AUTOSAR logo are registered trademarks.

Contents

1	Scope of Document	5

2	Conventions to be used	6

2.1	Document Conventions	6

2.2	Requirements Guidelines	6

2.3	Requirement structure	7

3	Acronyms and abbreviations	8

4	Requirements Specification	9

4.1	Functional Overview	9

4.2	Functional Requirements	18

4.2.1	Configuration and Initialization	18

4.2.2	Normal Operation	18

4.3	Non-Functional Requirements (Qualities)	21

5	Requirements Tracing	22

6	References	23

# Scope of Document

This document specifies requirements on the module DIO Driver.

Constraints

First scope for specification of requirements on basic software modules are systems which are not safety relevant. For this reason safety requirements are assigned to medium priority.

# Conventions to be used

## Document Conventions

The representation of requirements in AUTOSAR documents follows the table specified in [TPS_STDT_00078], see Standardization Template, chapter Support for Traceability ([1]).

The verbal forms for the expression of obligation specified in [TPS_STDT_00053] shall be used to indicate requirements, see Standardization Template, chapter Support for Traceability ([1]).

## Requirements Guidelines

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as follows.

Note that the requirement level of the document in which they are used modifies the force of these words.

- MUST: This word, or the adjective "LEGALLY REQUIRED", means that the defi- nition is an absolute requirement of the specification due to legal issues.

- MUST NOT: This phrase, or the phrase "MUST NOT", means that the definition is an absolute prohibition of the specification due to legal issues.

- SHALL: This phrase, or the adjective "REQUIRED", means that the definition is an absolute requirement of the specification.

- SHALL NOT: This phrase means that the definition is an absolute prohibition of the specification.

- SHOULD: This word, or the adjective "RECOMMENDED", means that there may exist valid reasons in particular circumstances to ignore a particular item, but the

full implications must be understood and carefully weighed before choosing a different course.

- SHOULD NOT: This phrase, or the phrase "NOT RECOMMENDED", means that there may exist valid reasons in particular circumstances when the particular be-

havior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- MAY: This word, or the adjective "OPTIONAL", means that an item is truly op- tional. One vendor may choose to include the item because a particular market- place requires it or because the vendor feels that it enhances the product while

another vendor may omit the same item.

An implementation, which does not include a particular option, SHALL be prepared to interoperate with another implementation, which does include the option, though perhaps with reduced functionality. In the same vein an implementation, which does include a particular option, SHALL be prepared to interoperate with another implemen- tation, which does not include the option (except, of course, for the feature the option provides.)

## Requirement structure

Mỗi chương dành riêng cho mô-đun đều chứa một mô tả chức năng ngắn gọn về Mô-đun Phần mềm Cơ bản (Basic Software Module). Các yêu cầu cùng loại trong mỗi chương được nhóm lại dưới các tiêu đề sau (khi áp dụng):

Yêu cầu chức năng

- Cấu hình (những thành phần nào của mô-đun cần có khả năng cấu hình)

- Khởi tạo (Initialisation)

- Vận hành bình thường (Normal Operation)

- Vận hành khi tắt hệ thống (Shutdown Operation)

- Vận hành khi có lỗi (Fault Operation)

- ...

Yêu cầu phi chức năng (Non-Functional Requirements)

- Yêu cầu thời gian/định thời (Timing Requirements)

- Mức sử dụng tài nguyên (Resource Usage)

- Tính dễ sử dụng (Usability)

- Đầu ra cho các gói công việc khác (WPs) (ví dụ: Mẫu mô tả – Description Templates, công cụ – Tooling,...)

- ...

# Acronyms and abbreviations

The glossary below includes acronyms and abbreviations relevant to DIO Driver that are not included in the AUTOSAR Glossary [2].

| Abbreviation / Acronym: | Description: |
| --- | --- |
| CS | Chip select |
| DIO | Digital Input Output |
| ECU | Electric Control Unit |
| EOL | End Of Line Often used in the term ’EOL Programming’ or ’EOL Configuration’ |
| ICU | Interrupt Capture Unit |
| MAL | Old name of Microcontroller Abstraction Layer (replaced by MCAL because ’MAL’ is a french term meaning ’bad’) |
| MCAL | Microcontroller Abstraction Layer |
| MCU | Microcontroller Unit |
| MMU | Memory Management Unit |
| Master | A device controlling other devices (slaves, see below) |
| Slave | A device being completely controlled by a master device |
| NMI | Non maskable interrupt |
| OS | Operating System |
| PLL | Phase Locked Loop |
| PWM | Pulse Width Modulation |
| RX | Reception (in the context of bus communication) |
| SPAL | The name of this working group |
| SFR | Special Function Register |
| RTE | Runtime environment |
| WP | Work Package |
| STD | Standard |
| REQ | Requirement |
| UNINIT | Uninitialized (= not initialized) |

Table 3.1: Acronyms and abbreviations used in the scope of this Document

# Requirements Specification

Chương này mô tả tất cả các yêu cầu chi phối công việc định nghĩa Trình điều khiển DIO.

## Functional Overview

Trình điều khiển DIO cung cấp khả năng truy cập đọc và ghi dựa trên cổng (port) và kênh (channel) tới các cổng I/O mục đích chung (general purpose I/O) nội bộ. Hành vi đọc và ghi là không đệm (unbuffered). Hành vi cơ bản của trình điều khiển này là đồng bộ (synchronous).

1) “đọc/ghi theo port và channel” - Channel (kênh) = 1 chân I/O (một bit). Ví dụ: chân P1_3 (Port 1, bit 3) là một Dio_ChannelType. - Port (cổng) = một nhóm nhiều chân (một thanh ghi port gồm nhiều bit). Ví dụ: Port 1 gồm các bit 0..15 (hoặc 0..31 tùy MCU) là Dio_PortType. Vì vậy DIO Driver cho phép: - Đọc/ghi 1 chân: Dio_ReadChannel(), Dio_WriteChannel() - Đọc/ghi cả port (nhiều bit một lần): Dio_ReadPort(), Dio_WritePort() - (thường có thêm) đọc/ghi theo mask/group: Dio_ReadChannelGroup(), Dio_WriteChannelGroup() 2) “read/write behavior is unbuffered” (không đệm) Nghĩa là driver không lưu giá trị vào bộ nhớ đệm nội bộ rồi mới áp ra chân. - Khi bạn gọi Read → driver đọc trực tiếp từ thanh ghi phần cứng (Input Data Register). - Khi bạn gọi Write → driver ghi trực tiếp xuống thanh ghi phần cứng (Output Data Register / Set-Reset Register). Hệ quả thực tế: - Giá trị trả về khi đọc là trạng thái tức thời của chân/port tại thời điểm gọi hàm. - Không có cơ chế “xếp hàng”, “cache”, “flush” như một số driver truyền thông. - Nếu chân bị phần cứng/ngoại vi khác tác động (ví dụ pin mux sang chức năng khác, hoặc bị pull-up/pull-down, hoặc có ngoại vi chiếm chân), kết quả read/write sẽ phản ánh trạng thái thực của phần cứng, không phải “giá trị bạn nhớ là đã ghi”. 3) “basic behavior is synchronous” (đồng bộ) Nghĩa là các API đọc/ghi của DIO: - thực hiện xong ngay trong lúc hàm đang chạy và - trả kết quả ngay (đối với read) hoặc ghi xong ngay (đối với write), - không có cơ chế bất đồng bộ kiểu “gọi xong rồi trả về, xử lý sau, báo completion bằng callback/interrupt”. Với DIO điều này hợp lý vì thao tác chỉ là đọc/ghi thanh ghi, thời gian rất ngắn.

Các thuật ngữ sau được sử dụng trong trình điều khiển DIO:

| Expression | Explanation |
| --- | --- |
| DIO channel | Biểu diễn một chân vào/ra số mục đích chung (general-purpose) đơn lẻ. |
| DIO port | Biểu diễn nhiều kênh DIO được nhóm lại theo phần cứng và có thể truy cập đồng bộ (thường được điều khiển bởi một thanh ghi phần cứng). Ví dụ: Cổng A (8 bit) của Freescale HC08 |
| DIO channel group | Biểu diễn nhiều kênh DIO liền kề được biểu diễn bởi một nhóm logic. Một nhóm kênh DIO là một tập con của một cổng DIO, có thể truy cập đồng bộ. Ví dụ: Các chân cổng 2..6 của một cổng 8 bit. |

1) DIO channel (Kênh DIO) = 1 chân I/O (1 bit) - Bản chất phần cứng: một chân GPIO vật lý trên MCU, tương ứng 1 bit trong thanh ghi port. - Bạn làm được gì: đọc/ghi một mức logic của đúng chân đó: 0/1 (LOW/HIGH). - Ví dụ (Port A 8-bit): - PA0, PA1, … PA7 là các channel. - Khi gọi Dio_WriteChannel(PA3, STD_HIGH) thì driver sẽ tác động đúng bit 3 của port A. Lưu ý: Channel chỉ “hợp lệ” để điều khiển bằng DIO khi chân đó đang được cấu hình đúng chế độ GPIO (thường do PORT driver cấu hình mux/direction trước). 2) DIO port (Cổng DIO) = 1 cụm nhiều chân chung 1 thanh ghi (n bit) - Bản chất phần cứng: một “cổng” gồm nhiều chân (8/16/32 bit…) được điều khiển/đọc qua một hoặc vài thanh ghi phần cứng chung. - “Accessible synchronously” nghĩa là gì ở đây? - Bạn có thể đọc/ghi cả cụm bit “một lần” thông qua 1 thao tác đọc/ghi thanh ghi. - Ví dụ: đọc PORTA_IN trả về ngay 8 bit trạng thái PA0..PA7 trong cùng một lần đọc. - Bạn làm được gì: đọc/ghi toàn bộ port: - Dio_ReadPort(PortA) → trả về giá trị kiểu bitmask (ví dụ 0b1010_0110). - Dio_WritePort(PortA, 0x5A) → ghi đồng thời 8 bit. Lưu ý quan trọng: Ghi port sẽ tác động tất cả các bit của port đó. Nếu trên port có bit bạn không muốn đổi (đang dùng cho mục đích khác), thì không nên dùng WritePort mà dùng ChannelGroup hoặc WriteChannel (hoặc thao tác mask). 3) DIO channel group (Nhóm kênh DIO) = một “dải bit liền kề” trong cùng 1 port - Bản chất: là một tập con của 1 port, gồm nhiều bit liền kề (adjoining) và được mô tả bởi: - port (thuộc port nào), - mask (những bit nào thuộc group), - offset (dịch bao nhiêu bit để đưa về LSB khi đọc/ghi). - Tại sao cần Channel Group? - Khi bạn muốn điều khiển/đọc một cụm tín hiệu nằm trên các bit liền nhau (ví dụ 5-bit data bus, DIP switch, địa chỉ, trạng thái…), mà: - không muốn đụng tới các bit khác trong port, - và muốn thao tác “gọn” hơn so với đọc/ghi từng channel. - Ví dụ “pins 2..6 của port 8-bit”: - Group = PA2..PA6 (5 bit) - mask = 0b0111_1100 (0x7C) - offset = 2 - Khi đọc: - đọc port A được PORTA = xxxx_xxxx - lấy PORTA & mask rồi dịch phải offset → ra giá trị 0..31 (5 bit). - Khi ghi: - driver chỉ cập nhật các bit trong mask (các bit khác giữ nguyên), thường theo kiểu: - đọc port hiện tại → clear vùng mask → OR dữ liệu đã dịch vào → ghi lại. Vì là “subset của 1 port”, channel group không được phép gom các chân từ nhiều port khác nhau (ví dụ PA7 + PB0 là không hợp lệ cho 1 group).

DIO CHANNEL GROUP 1. DIO channel group là gì? DIO channel group là một nhóm các kênh DIO (các chân GPIO) nằm trong cùng một DIO port và thường liền kề nhau (adjoining), để bạn có thể đọc/ghi cả nhóm trong một lần (thông qua thao tác đọc/ghi thanh ghi port) mà không làm ảnh hưởng các bit khác trong port. Nói cách khác: - Port = cả thanh ghi (ví dụ 8 bit/16 bit/32 bit). - Channel group = một phần của thanh ghi đó (một “miếng” bit), ví dụ bit 2..6. 2. Tại sao AUTOSAR cần Channel Group? Vì rất hay gặp trường hợp tín hiệu được nối theo cụm bit: - Một dãy DIP switch (5 bit) - Một bus dữ liệu nhỏ (4–8 bit) - Một cụm chân đọc trạng thái cảm biến/digital input - Một cụm chân điều khiển LED/relay theo bit Nếu dùng ReadChannel/WriteChannel thì phải thao tác từng chân → dài dòng và khó tối ưu. Nếu dùng ReadPort/WritePort thì lại đụng toàn bộ port → dễ làm “đổi nhầm” các bit khác đang dùng cho chức năng khác. => Channel group là lựa chọn “đúng”: đọc/ghi được cụm bit mong muốn, giữ nguyên phần còn lại. 3. Channel Group được mô tả bằng gì? Trong AUTOSAR, một channel group thường được định nghĩa bởi 3 thông tin chính: - Port: nhóm này thuộc port nào (ví dụ Port A) - Mask: những bit nào trong port thuộc group - Offset: số bit dịch để “đưa nhóm về bit thấp nhất” khi trả dữ liệu Ví dụ group “pins 2..6” trong port 8-bit: - Bit thuộc group: 2,3,4,5,6 - Mask = 0b0111_1100 = 0x7C - Offset = 2 4. Cách đọc Channel Group hoạt động như thế nào? Giả sử port A hiện đang có giá trị: - PORTA = 0b1011_0110 (0xB6) Group là bit 2..6 (mask 0x7C): - Lấy phần group: - PORTA & mask = 0b1011_0110 & 0b0111_1100 = 0b0011_0100 - Dịch phải offset=2 để đưa về LSB: - (PORTA & mask) >> 2 = 0b0011_0100 >> 2 = 0b0000_1101 (= 13) => Dio_ReadChannelGroup() sẽ trả về giá trị 13 (tức 5-bit data của pins 2..6). Điểm hay: bạn không cần quan tâm nó nằm ở bit 2..6; bạn nhận được một số “gọn” từ 0..31. 5. Cách ghi Channel Group hoạt động như thế nào? Giả sử bạn muốn ghi giá trị 0b10010 (= 18) vào group bit 2..6, nhưng không được làm đổi các bit khác (bit 0,1,7). Nguyên tắc an toàn thường là (về mặt logic): - Đọc port hiện tại: old = PORTA - Xóa vùng group: old_cleared = old & ~mask - Đặt dữ liệu mới vào đúng vị trí: - new_bits = (value << offset) & mask - Ghép lại: PORTA = old_cleared ¦ new_bits Vậy nhóm bit được cập nhật, còn các bit ngoài mask giữ nguyên. Lưu ý: một số MCU có thanh ghi set/reset riêng (atomic) giúp tránh vấn đề “read-modify-write” khi có ngắt; nhưng về mặt ý nghĩa của channel group trong spec thì vẫn là “chỉ tác động vùng mask”. “Accessible synchronously” nghĩa là gì với Channel Group? Nghĩa là group này nằm trong một port có thể đọc/ghi qua một thanh ghi nên thao tác group thực chất là: - đọc một lần thanh ghi port → xử lý mask/shift → trả kết quả - hoặc read-modify-write trên cùng port để ghi Không phải kiểu “bất đồng bộ” hay cần callback. 6. Vì sao spec nhấn mạnh “adjoining” (liền kề)? Vì khi nhóm bit liền kề, bạn có thể biểu diễn nó như một số nhị phân liên tục và dùng offset để “nén” về LSB. Ví dụ bit 2..6 nén thành 5-bit từ bit0..4. Nếu các bit rời rạc (ví dụ bit 0,2,5) thì: - vẫn có thể dùng mask, nhưng offset không đủ để “nén” thành một số liên tục đẹp (sẽ có lỗ hổng bit). - AUTOSAR thường định hướng channel group là “một dải” để thao tác như bus dữ liệu. 7. Khi nào bạn nên dùng Channel Group? - Khi tín hiệu là bus/nhóm bit và bạn muốn: - đọc/ghi nhanh, - code gọn, - không ảnh hưởng các bit khác trên cùng port. Không nên dùng nếu: - các chân nằm ở khác port, - hoặc bạn chỉ cần 1 chân (dùng channel), - hoặc bạn muốn set/reset từng bit độc lập và an toàn ngắt (tùy MCU, đôi khi dùng write channel + atomic set/reset tốt hơn). Nếu bạn gửi thêm đoạn trong spec về Dio_ReadChannelGroup / Dio_WriteChannelGroup (hoặc cấu trúc Dio_ChannelGroupType), mình sẽ giải thích đúng theo API và chỉ rõ mask/offset trong định nghĩa kiểu dữ liệu đó.

ĐỌC CHANNEL GROUP 1) PORTA là 8 ô (8 chân) Mình viết theo thứ tự từ trái sang phải: B7; B6; B5; B4; B3; B2; B1; B0. Ví dụ PORTA đang là: 1; 0; 1; 1; 0; 1; 1; 0. 2) Channel group b2..b6 nghĩa là bạn chỉ quan tâm 5 ô này Group b2..b6 là: B7; B6; B5; B4; B3; B2; B1; B0. Giá trị là: 1; 0; 1; 1; 0; 1; 1; 0. Trong ví dụ trên, 5 ô đó có giá trị: - b6 = 0 - b5 = 1 - b4 = 1 - b3 = 0 - b2 = 1 => group = [0][1][1][0][1] Đến đây là bản chất của “đọc group” rồi. Bạn đang lấy đúng 5 bit đó thôi. 3) Vậy MASK dùng để làm gì? Mask là một số nhị phân dùng để “chọn đúng các bit thuộc group”. - Bit nào thuộc group → mask đặt 1 - Bit nào không thuộc group → mask đặt 0 MASK chỉ là “băng keo che” những ô bạn không quan tâm. Mask cho b2..b6 sẽ che b7, b1, b0 và chỉ để hở b6..b2: MASK = [0][1][1][1][1][1][0][0] 0; 1; 1; 1; 1; 1; 0; 0. Khi bạn “AND” với mask, nghĩa là: - Ô nào mask = 0 → bị che → thành 0 - Ô nào mask = 1 → được giữ nguyên Kết quả sau “cắt” sẽ là: PORTA: [1] [0] [1] [1] [0] [1] [1] [0] MASK : [0] [1] [1] [1] [1] [1] [0] [0] KQ : [0] [0] [1] [1] [0] [1] [0] [0] Bạn thấy không: chỉ còn vùng giữa b6..b2, còn lại thành 0. 4) OFFSET dùng để làm gì? Sau khi “cắt”, các bit group vẫn đang nằm ở vị trí b2..b6 (lệch sang trái). Nhưng khi người ta đọc group, họ muốn nhận một số “gọn” bắt đầu từ bit0. Vì group bắt đầu ở b2, nên bạn phải dịch phải 2 ô để kéo nó về sát phải. KQ sau cắt: [0] [0] [1] [1] [0] [1] [0] [0] Lấy kết quả sau khi cắt này Dịch phải 2: [0] [0] [0] [0] [1] [1] [0] [1] Bây giờ 5 bit group đã nằm ở phía phải và chính là: [1][1][0][1] (thực tế là 5 bit từ b2..b6: 01101) ✅ Nói đơn giản: OFFSET = 2 vì group bắt đầu ở b2, nên kéo về b0 cần dịch 2 lần. Cách nhớ SIÊU DỄ - MASK = khoanh vùng (giữ lại đúng các bit thuộc group) - OFFSET = kéo vùng đó về sát phải để trả ra số gọn

GHI (WRITE) CHANNEL GROUP Giả sử: - PORTA (8 bit) hiện tại là: B7; B6; B5; B4; B3; B2; B1; B0. Ví dụ PORTA đang là: 1; 0; 1; 1; 0; 1; 1; 0. - Bạn muốn ghi vào group b2..b6 (5 bit). - Mask cho b2..b6: MASK = [0][1][1][1][1][1][0][0] 0; 1; 1; 1; 1; 1; 0; 0. - Offset = 2 (vì group bắt đầu ở b2). Bạn muốn ghi value = 10010 (5 bit) vào group b2..b6. Mục tiêu khi ghi ✅ Chỉ thay đổi b2..b6 ✅ Giữ nguyên b7, b1, b0 Tức là: - b7 giữ nguyên = 1 - b1 giữ nguyên = 1 - b0 giữ nguyên = 0 Còn b2..b6 sẽ thành theo value mới. Quy trình ghi chuẩn (3 bước) Bước 1: “Xóa vùng group” trên PORTA cũ (nhưng giữ bit ngoài vùng) Ta tạo ~MASK (phần bù của mask) để chỉ giữ vùng ngoài group: ~MASK = [1 0 0 0 0 0 1 1] Giờ lấy phần “ngoài group” của PORTA cũ: PORTA_cũ & ~MASK PORTA_cũ: [1 0 1 1 0 1 1 0] ~MASK : [1 0 0 0 0 0 1 1] Kết quả : [1 0 0 0 0 0 1 0] 👉 Nghĩa là: - b7 vẫn giữ 1 - b1 vẫn giữ 1 - b0 vẫn giữ 0 - vùng b2..b6 đã bị xóa về 0 để chuẩn bị “nhét cái mới vào”. Bước 2: “Đưa value lên đúng vị trí b2..b6” Value của bạn là 5 bit: value = [b4 b3 b2 b1 b0] = [1 0 0 1 0] (tức 10010) Nhưng group nằm ở b2..b6, nên ta phải dịch trái 2: value << 2 → đẩy value vào vùng b2..b6 10010 << 2 = 1001000 (7 bit), trong 8-bit là: [0 1 0 0 1 0 0 0] Giờ AND với MASK để đảm bảo chỉ nằm trong vùng group: (value << 2) & MASK value<<2: [0 1 0 0 1 0 0 0] MASK : [0 1 1 1 1 1 0 0] Kết quả : [0 1 0 0 1 0 0 0] (vẫn vậy, vì đã nằm đúng vùng) 👉 Đây là “miếng bit mới” chỉ dành cho b2..b6. Bước 3: Ghép lại (OR) Cuối cùng: PORTA_mới = (PORTA_cũ & ~MASK) ¦ ((value<<offset) & MASK) Ta OR 2 phần lại: Phần ngoài group: [1 0 0 0 0 0 1 0] Phần group mới : [0 1 0 0 1 0 0 0] PORTA_mới : [1 1 0 0 1 0 1 0] ✅ Kết quả: - b7 giữ nguyên (1) - b1 giữ nguyên (1) - b0 giữ nguyên (0) - b2..b6 đã đổi theo value (10010) Nhớ 1 câu thôi là đủ Ghi group = Xóa vùng cũ trong group + đặt vùng mới vào + giữ nguyên phần ngoài group. Hình trực quan phần ghi Bạn có thể nhìn hình này, nó đúng đúng 3 bước ở trên: dio_channel_group_write.png Nếu bạn muốn mình giải theo đúng API AUTOSAR (Dio_WriteChannelGroup) với biến Dio_ChannelGroupType {mask, offset, port} mình cũng làm được, nhưng phần trên là “bản chất” để bạn hiểu chắc chắn.

## Functional Requirements

### Configuration and Initialization

Việc cấu hình và khởi tạo cấu trúc cổng (port structure) không thuộc phạm vi của trình điều khiển DIO. Việc này được thực hiện bởi trình điều khiển Port (xem [SRS_Port_12001] Cấu hình các thuộc tính của chân cổng (port pin) trong [3]).

[SRS_Dio_12355] Symbolic names shall be configured [

| Description: | The DIO driver shall allow the static configuration of the following symbolic names: - DIO channel names - DIO channel group names - DIO port names |
| --- | --- |
| Rationale: | Provide human readable symbolic names for DIO channels |
| Use Case: | – |
| Dependencies: | – |
| Supporting Material: | – |

♩(RS_BRF_01024)

### Normal Operation

[SRS_Dio_12003] The DIO Driver shall provide a service that writes a data word to the assigned DIO port [

| Description: | The DIO Driver shall provide a service that writes a data word to the assigned DIO port. The operation shall be unbuffered. There shall be no influence to the input functionality of the port. |
| --- | --- |
| Rationale: | Basic functionality |
| Use Case: | Write access to an entire DIO port. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12004] The DIO Driver shall provide a service that writes a selectable number of adjoining bits to an assigned part of a DIO port [

| Description: | The DIO Driver shall provide a service that writes a selectable number of adjoining bits to an assigned part of a DIO port. The operation shall be unbuffered. |
| --- | --- |
| Rationale: | Allow for simultaneous setting of a group of DIO channels of a DIO port that has multiple external assignments. |
| Use Case: | Write access to DIO ports with multiple assignments. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12005] The DIO Driver shall provide a service for write access to single DIO channels [

| Description: | The DIO Driver shall provide a service for write access to single DIO channels (specific port pins). |
| --- | --- |
| Rationale: | Efficient handling of single DIO channels. |
| Use Case: | Write access to a particular DIO channel (port pin). |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12006] The DIO Driver shall provide a service for reading a data word from the assigned DIO port [

| Description: | The DIO Driver shall provide a service for reading a data word from the assigned DIO port. The operation shall be unbuffered. There shall be no influence to the output functionality of the port. |
| --- | --- |
| Rationale: | Basic functionality |
| Use Case: | Read access to an entire DIO port. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12007] The DIO Driver shall provide a service for reading a selectable number of adjoining bits from an assigned part of a DIO port [

| Description: | The DIO Driver shall provide a service for reading a selectable number of adjoining bits from an assigned part of a DIO port. The operation shall be unbuffered. |
| --- | --- |
| Rationale: | Basic functionality |
| Use Case: | Read access to DIO ports with multiple assignments. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12008] The DIO Driver shall provide a service for reading one bit of an assigned DIO channel [

| Description: | The DIO Driver shall provide a service for reading one bit of an assigned DIO channel (specific port pin). The operation shall be unbuffered. |
| --- | --- |
| Rationale: | Efficient handling of single DIO channels. |
| Use Case: | Read access to a particular DIO channel. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12900] The DIO Driver shall provide a service to flip [

| Description: | The DIO Driver shall provide a service to flip (change from 1 to 0 or from 0 to 1) one bit of an assigned DIO channel (specific port pin) and return the level of the channel after flip. The operation shall be unbuffered. |
| --- | --- |
| Rationale: | Efficient handling of single DIO channels. |
| Use Case: | Read & write access to a particular DIO channel to flip the level. |
| Dependencies: | [SRS_Dio_12352] General read/write behavior |
| Supporting Material: | – |

♩(RS_BRF_01864)

[SRS_Dio_12352] The DIO driver shall allow reading from and writing to DIO ports, channel groups and channels [

| Description: | The DIO driver shall allow reading from and writing to DIO ports, channel groups and channels regardless of the configuration of their direction. If a channel configured as input is written to, the value shall be written to the output register, but does not appear on the physical port pin. If a channel configured as output is read, the value of the real pin level is read, if supported by hardware. Otherwise, the value of the port output register is read. |
| --- | --- |
| Rationale: | Simplify implementation of all DIO read and write services. Allow readback of output pins. Allow presetting of output values before switching the port pin to output direction. |
| Use Case: | See rationale. |
| Dependencies: | – |
| Supporting Material: | – |

♩(RS_BRF_01864)

## Non-Functional Requirements (Qualities)

[SRS_Dio_12424] Provide atomicity of DIO access [

| Description: | All re-entrant functions of the DIO Driver shall perform the following access actions in an atomic way: - DIO ports - DIO channels DIO channel groups |
| --- | --- |
| Rationale: | Avoid data integrity problems within concurrent access of DIO Driver API functions. |
| Use Case: | A specific microcontroller (or a specific compiler) does not provide atomic access to single port pins. For that reason, the implementation has to use read-modify-write operations on the whole port. Concurrent access to pins of the same port will lead to data integrity problems if concurrent access is not blocked. |
| Dependencies: | – |
| Supporting Material: | – |

♩(RS_BRF_01864)

# Requirements Tracing

The following table references the features specified in [4] and links to the fulfillments of these.

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [RS_BRF_01024] | AUTOSAR shall provide naming rules for public symbols | [SRS_Dio_12355] |
| [RS_BRF_01864] | AUTOSAR microcontroller abstraction shall provide mapping of I/O signals to digital I/O ports | [SRS_Dio_12003] [SRS_Dio_12004] [SRS_Dio_12005] [SRS_Dio_12006] [SRS_Dio_12007] [SRS_Dio_12008] [SRS_Dio_12352] [SRS_Dio_12424] [SRS_Dio_12900] |

Table 5.1: RequirementsTracing

# References

- Standardization Template AUTOSAR_FO_TPS_StandardizationTemplate

- Glossary AUTOSAR_FO_TR_Glossary

- Requirements on Port Driver AUTOSAR_CP_SRS_PortDriver

- Requirements on AUTOSAR Features AUTOSAR_CP_RS_Features