| Document Title | Specification of DIO Driver |
| --- | --- |
| Document Owner | AUTOSAR |
| Document Responsibility | AUTOSAR |
| Document Identification No | 20 |

| Document Status | published |
| --- | --- |
| Part of AUTOSAR Standard | Classic Platform |
| Part of Standard Release | R23-11 |

| Document Change History |  |  |  |
| --- | --- | --- | --- |
| Date | Release | Changed by | Description |
| 2023-11-23 | R23-11 | AUTOSAR Release Management | - Editorial changes |
| 2022-11-24 | R22-11 | AUTOSAR Release Management | - Minor corrections / clarifications / editorial changes |
| 2021-11-25 | R21-11 | AUTOSAR Release Management | - Cleaned return codes |
| 2020-11-30 | R20-11 | AUTOSAR Release Management | - Cleaned error classification |
| 2019-11-28 | R19-11 | AUTOSAR Release Management | - MCALMulticoreDistribution (CONC_639) - Changed Document Status from Final to published |
| 2018-10-31 | 4.4.0 | AUTOSAR Release Management | - Introduced MaskedWritePort API |
| 2017-12-08 | 4.3.1 | AUTOSAR Release Management | - Removed unused artifacts - Editorial changes |

q

| 2016-11-30 | 4.3.0 | AUTOSAR Release Management | - Removed [SWS_Dio_00065] - Replaced content of "7.6.2 Runtime Errors" by "There are no runtime errors." - Replaced content of "7.6.3 Transient Faults" by "There are no transient faults - Removed the definition of the "configuration variants" from 10.1.1 - Changed Figure 2: Include File Structure |
| --- | --- | --- | --- |
| 2015-07-31 | 4.2.2 | AUTOSAR Release Management | - DET Renaming and Extension Incorporation - Changed DioChannelId, DioPortId precompile configuration |
| 2014-10-31 | 4.2.1 | AUTOSAR Release Management | - DIO: ReadChannelGroup / WriteChannelGroup pointer parameters. Provided support for Link time only. - The generation of link-time parameters aggregated by a postBuildChangeable container may not be possible. Reference to SWS_BSW_00380 is removed. |
| 2013-10-31 | 4.1.2 | AUTOSAR Release Management | - Formalization of Service Interfaces - Revised return values of Service Interfaces - Editorial changes - Removed chapter(s) on change documentation |
| 2013-03-15 | 4.1.1 | AUTOSAR Administration | - Updated chapter 10 for ’Scope’ values - Changed ’MemMap.h’ to ’Dio_MemMap.h’ - Added Subchapter 3.x - Requirement IDs changed to new format |
| 2011-12-22 | 4.0.3 | AUTOSAR Administration | - Removed Dem.h from [SWS_Dio_00171] and added new requirement [SWS_Dio_00194] |

Δ

q

| 2010-09-30 | 3.1.5 | AUTOSAR Administration | - Added a new API "Dio_LevelType Dio_FlipChannel(Dio_ChannelType ChannelId)" to flip (change from 1 to 0 or from 0 to 1) the level of a channel and return the level of the channel after flip. - Removed requirement DIO174 and rephrased [SWS_Dio_00106]. - Added requirements [SWS_DIO_00188] and [SWS_Dio_00189], to report DET error DIO_E_PARAM_POINTER from Dio_GetVersionInfo. |
| --- | --- | --- | --- |
| 2010-02-02 | 3.1.4 | AUTOSAR Administration | - Clarification of [SWS_Dio_00014] - DioVersionInfoApi added to DIO071 - Clean up of configuration parameters and header file inclusion structure - Legal disclaimer revised |
| 2008-08-13 | 3.1.1 | AUTOSAR Administration | - Legal disclaimer revised |
| 2007-12-21 | 3.0.1 | AUTOSAR Administration | - Harmonized initialization with MCAL modules - Optional inclusion of the DEM header file - Added explanation on dependency between DIO_PORT_MASK and DIO_PORT_OFFSET - Document meta information extended - Small layout adaptations made |

Δ

q

Δ

| 2007-01-24 | 2.1.15 | AUTOSAR Administration | - File structure update - Removed BSW00324 - In the configuration where pre-compile and link time is possible the variant for pre-compile is now always "PC" and not "All variants". - Added Chapter 8.6 - Changes in referencing symbolic naming - Updated traceability matrix regarding SRS_BSW_00435 and SRS_BSW_00436 - Legal disclaimer revised - "Advice for users" revised - "Revision Information" added |
| --- | --- | --- | --- |
| 2006-05-16 | 2.0 | AUTOSAR Administration | - Major changes in chapter 10, Configuration specification - Structure of document changed partly - Readback support moved to PORT Driver |
| 2005-05-31 | 1.0 | AUTOSAR Administration | - Initial Release |

### Disclaimer

This work (specification and/or software implementation) and the material contained in it, as released by AUTOSAR, is for the purpose of information only. AUTOSAR and the companies that have contributed to it shall not be liable for any use of the work.

The material contained in this work is protected by copyright and other types of intel- lectual property rights. The commercial exploitation of the material contained in this work requires a license to such intellectual property rights.

This work may be utilized or reproduced without any modification, in any form or by any means, for informational purposes only. For any other purpose, no part of the work may be utilized or reproduced, in any form or by any means, without permission in writing from the publisher.

The work has been developed for automotive applications only. It has neither been developed, nor tested for non-automotive applications.

The word AUTOSAR and the AUTOSAR logo are registered trademarks.

# Contents

- Introduction and functional overview	8

- Acronyms and Abbreviations	10

- Related documentation	11

  - Input documents & related standards and norms	11

  - Related specification	11

- Constraints and assumptions	12

  - Limitations	12

  - Applicability to car domains	12

- Dependencies to other modules	13

  - Port Driver Module	13

  - Default Error Tracer	13

- Requirements Tracing	14

- Functional specification	16

  - General Behaviour	16

    - Background & Rationale	16

    - Requirements	16

  - Initialization	18

    - Background & Rationale	18

    - Requirements	18

  - Runtime Reconfiguration	18

    - Background & Rationale	18

    - Requirements	18

  - DIO Write Service	18

    - Background & Rationale	18

    - Requirements	18

      - DIO Channel Write Service	19

      - DIO Port Write Service	19

      - DIO Channel Group Write Service	19

      - DIO Masked Port Write Service	19

  - DIO Read Service	20

    - Background & Rationale	20

    - Requirements	20

      - DIO Channel Read Service	20

      - DIO Port Read Service	20

      - DIO Channel Group Read Service	20

      - DIO Readback of Output Pins	20

  - Error Classification	21

    - Development Errors	21

    - Runtime Errors	21

    - Transient Faults	21

    - Production Errors	21

    - Extended Production Errors	21

  - Security Events	22

- API specification	23

  - Imported types	23

  - Type definitions	23

    - Dio_ChannelType	23

    - Dio_PortType	24

    - Dio_ChannelGroupType	24

    - Dio_LevelType	25

    - Dio_PortLevelType	25

  - Function definitions	25

    - Dio_ReadChannel	26

    - Dio_WriteChannel	26

    - Dio_ReadPort	27

    - Dio_WritePort	28

    - Dio_ReadChannelGroup	29

    - Dio_WriteChannelGroup	29

    - Dio_GetVersionInfo	30

    - Dio_FlipChannel	31

    - Dio_MaskedWritePort	32

  - Callback notifications	32

  - Scheduled functions	32

  - Expected interfaces	33

    - Mandatory Interfaces	33

    - Optional Interfaces	33

- Sequence diagrams	34

  - Read a value from a digital I/O	34

  - Write a value to a digital I/O	34

- Configuration specification	35

  - Containers and configuration parameters	35

    - Variants	35

    - Dio	36

    - DioGeneral	36

    - DioPort	38

    - DioChannel	39

    - DioChannelGroup	41

    - DioConfig	43

  - Published Information	43

A  Not applicable requirements	44

# Introduction and functional overview

Tài liệu đặc tả này mô tả chức năng, API và cấu hình của mô-đun Phần mềm Cơ bản AUTOSAR DIO Driver.

Tài liệu đặc tả này chỉ áp dụng cho các driver dành cho các chân và cổng DIO trên chip.

DIO Driver cung cấp các dịch vụ để đọc và ghi dữ liệu tới / từ:

- DIO Channels (Pins)

- DIO Ports

- DIO Channel Groups

Hành vi của các dịch vụ này là đồng bộ (synchronous).

Mô-đun này hoạt động trên các chân và cổng đã được cấu hình bởi PORT driver cho mục đích này. Vì lý do đó, không có cấu hình và khởi tạo các cổng này trong DIO Driver.

Sơ đồ bên dưới xác định các chức năng của DIO Driver, và cấu trúc của PORT Driver và DIO Driver trong tầng phần mềm MCAL.

Figure 1.1: DIO Driver Structure and Integration

# Acronyms and Abbreviations

The glossary below includes acronyms and abbreviations relevant to the DIO Driver module that are not included in the [1, AUTOSAR glossary].

| Abbreviation / Acronym | Description |
| --- | --- |
| DIO channel | Represents a single general-purpose digital input / output pin |
| DIO port | Represents several DIO channels that are grouped by hardware (typically controlled by one hardware register). Example: Port A (8 bit) of Freescale HC08 |
| DIO channel group | Represents several adjoining DIO channels represented by a logical group. A DIO channel group shall belong to one DIO port. Example: Port pins 2..6 of an 8 bit port addressing a multiplexer |
| Physical Level (Input) | Two states possible: LOW / HIGH. A bit value ’0’ represents a LOW, a bit value ’1’ represents a HIGH. |
| Physical Level (Output) | Two states possible: LOW / HIGH. A bit value ’0’ represents a LOW, a bit value ’1’ represents a HIGH. |
| LSB | Least Significant Bit |
| MSB | Most Significant Bit |
| DIO | Digital Input Output |
| ID | Identifier |
| ADC | Analog to Digital Converter |
| SPI | Serial Peripheral Interface |
| PWM | Pulse Width Modulation |
| ICU | Input Capture Unit |
| DET | Default Error Tracer |
| DEM | Diagnostic Event Manager |

Table 2.1: Acronyms and abbreviations used in the scope of this Document

# Related documentation

## Input documents & related standards and norms

- Glossary AUTOSAR_FO_TR_Glossary

- General Specification of Basic Software Modules AUTOSAR_CP_SWS_BSWGeneral

## Related specification

AUTOSAR provides a General Specification on Basic Software modules [2, SWS BSW General], which is also valid for DIO Driver.

Thus, the specification SWS BSW General shall be considered as additional and re- quired specification for DIO Driver.

# Constraints and assumptions

## Limitations

No limitations.

## Applicability to car domains

No restrictions.

# Dependencies to other modules

## Port Driver Module

Many ports and port pins are assigned by the PORT Driver Module to various function- alities as for example:

- General purpose I/O

- ADC

- SPI

- PWM

[SWS_Dio_00061] [The Dio module shall not provide APIs for overall configuration and initialization of the port structure which is used in the Dio module. These actions are done by the PORT Driver Module.♩()

[SWS_Dio_00063] [The Dio module shall adapt its configuration and usage to the microcontroller and ECU.♩()

[SWS_Dio_00102] [The Dio module’s user shall only use the Dio functions after the Port Driver has been initialized. Otherwise the Dio module will exhibit undefined be- havior.♩()

## Default Error Tracer

[SWS_Dio_00194] [Dio.c shall include Det.h if detection of development error (DET) is enabled.♩(SRS_BSW_00350)

# Requirements Tracing

This chapter refers to input requirements specified in the SRS documents (Software Requirements Specifications) that are applicable for this software module.

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_BSW_00323] | All AUTOSAR Basic Software Modules shall check passed API parameters for validity | [SWS_Dio_00074] [SWS_Dio_00075] [SWS_Dio_00114] [SWS_Dio_00189] |
| [SRS_BSW_00344] | BSW Modules shall support link-time configuration | [SWS_Dio_00001] [SWS_Dio_00002] |
| [SRS_BSW_00350] | All AUTOSAR Basic Software Modules shall allow the enabling/ disabling of detection and reporting of development errors. | [SWS_Dio_00194] |
| [SRS_BSW_00396] | The Basic Software Module specifications shall specify the supported configuration classes for changing values and multiplicities for each parameter/container | [SWS_Dio_00129] |
| [SRS_BSW_00411] | All AUTOSAR Basic Software Modules shall apply a naming rule for enabling/disabling the existence of the API | [SWS_Dio_00139] |
| [SRS_Dio_12003] | The DIO Driver shall provide a service that writes a data word to the assigned DIO port | [SWS_Dio_00004] [SWS_Dio_00007] [SWS_Dio_00034] [SWS_Dio_00035] [SWS_Dio_00051] [SWS_Dio_00053] [SWS_Dio_00089] [SWS_Dio_00105] [SWS_Dio_00108] [SWS_Dio_00200] [SWS_Dio_00201] [SWS_Dio_00202] [SWS_Dio_00203] [SWS_Dio_00204] |
| [SRS_Dio_12004] | The DIO Driver shall provide a service that writes a selectable number of adjoining bits to an assigned part of a DIO port | [SWS_Dio_00008] [SWS_Dio_00021] [SWS_Dio_00039] [SWS_Dio_00040] [SWS_Dio_00051] [SWS_Dio_00056] [SWS_Dio_00089] [SWS_Dio_00090] [SWS_Dio_00091] |
| [SRS_Dio_12005] | The DIO Driver shall provide a service for write access to single DIO channels | [SWS_Dio_00006] [SWS_Dio_00028] [SWS_Dio_00029] [SWS_Dio_00051] [SWS_Dio_00079] [SWS_Dio_00089] [SWS_Dio_00127] [SWS_Dio_00128] |
| [SRS_Dio_12006] | The DIO Driver shall provide a service for reading a data word from the assigned DIO port | [SWS_Dio_00013] [SWS_Dio_00031] [SWS_Dio_00051] [SWS_Dio_00053] [SWS_Dio_00089] [SWS_Dio_00104] |
| [SRS_Dio_12007] | The DIO Driver shall provide a service for reading a selectable number of adjoining bits from an assigned part of a DIO port | [SWS_Dio_00014] [SWS_Dio_00021] [SWS_Dio_00037] [SWS_Dio_00051] [SWS_Dio_00056] [SWS_Dio_00089] [SWS_Dio_00092] [SWS_Dio_00093] |
| [SRS_Dio_12008] | The DIO Driver shall provide a service for reading one bit of an assigned DIO channel | [SWS_Dio_00011] [SWS_Dio_00027] [SWS_Dio_00051] [SWS_Dio_00089] [SWS_Dio_00127] [SWS_Dio_00128] |
| [SRS_Dio_12352] | The DIO driver shall allow reading from and writing to DIO ports, channel groups and channels | [SWS_Dio_00012] [SWS_Dio_00064] [SWS_Dio_00070] [SWS_Dio_00083] [SWS_Dio_00084] [SWS_Dio_00109] |
| [SRS_Dio_12355] | Symbolic names shall be configured | [SWS_Dio_00017] [SWS_Dio_00020] [SWS_Dio_00022] [SWS_Dio_00026] |

The table below lists links to specification items of the DIO driver SWS document, that satisfy the input requirements. Only functional requirements are referenced.

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_Dio_12424] | Provide atomicity of DIO access | [SWS_Dio_00005] [SWS_Dio_00060] |
| [SRS_Dio_12900] | The DIO Driver shall provide a service to flip | [SWS_Dio_00191] [SWS_Dio_00192] [SWS_Dio_00193] |
| [SRS_SPAL_12064] | All driver modules shall raise an error if the change of the operation mode leads to degradation of running operations | [SWS_Dio_00001] [SWS_Dio_00002] |
| [SRS_SPAL_12263] | The implementation of all driver modules shall allow the configuration of specific module parameter types at link time | [SWS_Dio_00017] [SWS_Dio_00020] [SWS_Dio_00022] |
| [SRS_SPAL_12448] | All driver modules shall have a specific behavior after a development error detection | [SWS_Dio_00074] [SWS_Dio_00075] [SWS_Dio_00114] [SWS_Dio_00119] |
| [SRS_SPAL_12461] | Specific rules regarding initialization of controller registers shall apply to all driver implementations | [SWS_Dio_00001] [SWS_Dio_00002] |
| [SRS_SPAL_12462] | The register initialization settings shall be published | [SWS_Dio_00001] [SWS_Dio_00002] |
| [SRS_SPAL_12463] | The register initialization settings shall be combined and forwarded | [SWS_Dio_00001] [SWS_Dio_00002] |

Table 6.1: RequirementsTracing

# Functional specification

## General Behaviour

### Background & Rationale

The DIO Driver abstracts the access to the microcontroller’s hardware pins. Further- more, it allows the grouping of those pins.

### Requirements

The Dio SWS shall define functions allowing

- Port-

- Channel-

- Channel-group -

-based read and write access to the internal general purpose I/O ports.

[SWS_Dio_00051] [The Dio module shall not buffer data when providing read and write services.

The Dio SWS shall define synchronous read / write services.♩(SRS_Dio_12003, SRS_- Dio_12004, SRS_Dio_12005, SRS_Dio_12006, SRS_Dio_12007, SRS_Dio_12008)

[SWS_Dio_00005] [The Dio module’s read and write services shall ensure for all ser- vices, that the data is consistent (Interruptible read-modify-write sequences are not allowed).♩(SRS_Dio_12424)

[SWS_Dio_00089] [Values used by the DIO Driver for the software level of Channels are either STD_HIGH or STD_LOW.♩(SRS_Dio_12003, SRS_Dio_12004, SRS_Dio_- 12005, SRS_Dio_12006, SRS_Dio_12007, SRS_Dio_12008)

[SWS_Dio_00128] [A general-purpose digital IO pin represents a DIO channel.♩

(SRS_Dio_12005, SRS_Dio_12008)

[SWS_Dio_00127] [The Port module shall configure a DIO channel as input or output ([SWS_Dio_00001] and [SWS_Dio_00002]).♩(SRS_Dio_12005, SRS_Dio_12008)

[SWS_Dio_00053] [In the DIO Driver, it shall be possible to group several DIO chan- nels by hardware (typically controlled by one hardware register) to represent a DIO port.♩(SRS_Dio_12003, SRS_Dio_12006)

Note: The single DIO channel levels inside a DIO port represent a bit in the DIO port value, depending on their position inside the port.

[SWS_Dio_00056] [A channel group is a formal logical combination of several adjoin- ing DIO channels within a DIO port.♩(SRS_Dio_12004, SRS_Dio_12007)

Figure 7.1: Schematic description of a ChannelGroup

The DIO Driver provides the following services:

- The Dio SWS shall define functions to modify the levels of output channels indi- vidually, for a port or for a channel group.

- The Dio SWS shall define functions to read the level of input and output (see [SWS_Dio_00083]) channels individually, for a port or for a channel group.

Figure 7.2: DIO Services

[SWS_Dio_00060] [All read and write functions of the Dio module shall be re-entrant.

Reason: The DIO Driver may be accessed by different upper layer handlers or drivers. These upper layer modules may access the driver concurrently.♩(SRS_Dio_12424)

[SWS_Dio_00026] [The configuration process for Dio module shall provide symbolic names for each configured DIO channel, port and group.♩(SRS_Dio_12355)

## Initialization

### Background & Rationale

Initialization of the hardware is done by the PORT Driver.

### Requirements

[SWS_Dio_00001] [The Dio module shall not provide an interface for initialization of the hardware. The Port Driver performs this.♩(SRS_BSW_00344, SRS_SPAL_12064, SRS_SPAL_12461, SRS_SPAL_12462, SRS_SPAL_12463)

## Runtime Reconfiguration

### Background & Rationale

Runtime reconfiguration is provided by the PORT Driver.

### Requirements

[SWS_Dio_00002] [The PORT driver shall provide the reconfiguration of the port pin direction during runtime.♩(SRS_BSW_00344, SRS_SPAL_12064, SRS_SPAL_12461, SRS_SPAL_12462, SRS_SPAL_12463)

## DIO Write Service

### Background & Rationale

The DIO Driver provides services to transfer data to the microcontroller’s pins.

### Requirements

[SWS_Dio_00064] [The Dio module’s write functions shall work on input and output channels.♩(SRS_Dio_12352)

[SWS_Dio_00070] [If a Dio write function is used on an input channel, it shall have no effect on the physical output level.♩(SRS_Dio_12352)

[SWS_Dio_00109] [If supported by hardware, the Dio module shall set / clear the output data latch of an input channel so that the required level is output from the pin when the port driver configures the pin as a DIO output pin.♩(SRS_Dio_12352)

[SWS_Dio_00119] [If development errors are enabled and an error occurred, the Dio module’s write functions shall NOT process the write command.♩(SRS_SPAL_12448)

### DIO Channel Write Service

[SWS_Dio_00006] [The Dio_WriteChannel function shall set the level of a single DIO channel to STD_HIGH or STD_LOW.♩(SRS_Dio_12005)

### DIO Port Write Service

[SWS_Dio_00007] [The Dio_WritePort function shall simultaneously set the levels of all output channels. A bit value ’0’ sets the corresponding channel to physical STD_- LOW, a bit value ’1’ sets the corresponding channel to physical STD_HIGH.♩(SRS_Dio_- 12003)

[SWS_Dio_00004] [The Dio_WritePort function shall ensure that the functionality of the input channels of that port is not affected.♩(SRS_Dio_12003)

### DIO Channel Group Write Service

[SWS_Dio_00008] [The Dio_WriteChannelGroup function shall simultaneously set an adjoining subset of DIO channels (channel group). A bit value ’0’ sets the corre- sponding channel to physical STD_LOW, a bit value ’1’ sets the corresponding channel

to physical STD_HIGH.♩(SRS_Dio_12004)

### DIO Masked Port Write Service

[SWS_Dio_00200] [The Dio_MaskedWritePort function shall simultaneously set the levels of the selected output channels.

Mask argument specifies which bits are selected: a bit value ’0’ means the corre- sponding channel is not selected, a bit value ’1’ means the corresponding channel is selected.

Level argument specifies the physical levels: a bit value ’0’ sets the corresponding channel to physical STD_LOW, a bit value ’1’ sets the corresponding channel to physical

STD_HIGH.♩(SRS_Dio_12003)

[SWS_Dio_00201] [The Dio_MaskedWritePort function shall ensure that the func- tionality of the input channels of that port is not affected.♩(SRS_Dio_12003)

## DIO Read Service

### Background & Rationale

The DIO Driver provides services to transfer data from the microcontroller’s pins.

### Requirements

[SWS_Dio_00012] [The Dio module’s read functions shall work on input and output channels.♩(SRS_Dio_12352)

### DIO Channel Read Service

[SWS_Dio_00011] [The Dio_ReadChannel function shall read the level of a single DIO channel.♩(SRS_Dio_12008)

### DIO Port Read Service

[SWS_Dio_00013] [The Dio_ReadPort function shall read the levels of all channels of one port. A bit value ’0’ indicates that the corresponding channel is physical STD_- LOW, a bit value ’1’ indicates that the corresponding channel is physical STD_HIGH.♩ (SRS_Dio_12006)

### DIO Channel Group Read Service

[SWS_Dio_00014] [The Dio_ReadChannelGroup function shall read the levels of a DIO channel group. A bit value ’0’ indicates that the corresponding channel is physical

STD_LOW, a bit value ’1’ indicates that the corresponding channel is physical STD_- HIGH.♩(SRS_Dio_12007)

### DIO Readback of Output Pins

[SWS_Dio_00083] [If the microcontroller supports the direct read-back of a pin value, the Dio module’s read functions shall provide the real pin level, when they are used on a channel which is configured as an output channel.♩(SRS_Dio_12352)

[SWS_Dio_00084] [If the microcontroller does not support the direct read-back of a pin value, the Dio module’s read functions shall provide the value of the output register, when they are used on a channel which is configured as an output channel.♩(SRS_- Dio_12352)

## Error Classification

Section "Error Handling" of the document [2] "General Specification of Basic Software Modules" describes the error handling of the Basic Software in detail. Above all, it constitutes a classification scheme consisting of five error types which may occur in BSW modules.

Based on this foundation, the following section specifies particular errors arranged in the respective subsections below.

### Development Errors

[SWS_Dio_91001] Definiton of development errors in module Dio [

| Type of error | Related error code | Error value |
| --- | --- | --- |
| Invalid channel requested | DIO_E_PARAM_INVALID_CHANNEL_ID | 0x0A |
| Invalid port requested | DIO_E_PARAM_INVALID_PORT_ID | 0x14 |
| Invalid channel group requested | DIO_E_PARAM_INVALID_GROUP | 0x1F |
| API service called with a NULL pointer | DIO_E_PARAM_POINTER | 0x20 |

♩()

### Runtime Errors

There are no runtime errors.

### Transient Faults

There are no transient faults.

### Production Errors

There are no production errors.

### Extended Production Errors

There are no extended production errors.

## Security Events

The module does not report security events.

# API specification

## Imported types

In this chapter all types included from the following modules are listed:

### [SWS_Dio_00131] Definition of imported datatypes of module Dio [

| Module | Header File | Imported Type |
| --- | --- | --- |
| Std | Std_Types.h | Std_ReturnType |
|  | Std_Types.h | Std_VersionInfoType |

♩()

## Type definitions

[SWS_Dio_00103] [The port width within the types defined for the DIO Driver shall be the size of the largest port on the MCU which may be accessed by the DIO Driver.♩()

### Dio_ChannelType

[SWS_Dio_00182] Definition of datatype Dio_ChannelType [

| Name | Dio_ChannelType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint |  |  |
| Range | This is implementation specific but not all values may be valid within the type. | – | Shall cover all available DIO channels |
| Description | Numeric ID of a DIO channel. |  |  |
| Available via | Dio.h |  |  |

♩()

[SWS_Dio_00015] [Parameters of type Dio_ChannelType contain the numeric ID of a DIO channel.♩()

[SWS_Dio_00180] [The mapping of the ID is implementation specific but not config- urable.♩()

[SWS_Dio_00017] [For parameter values of type Dio_ChannelType, the Dio’s user shall use the symbolic names provided by the configuration description.

Furthermore, [SWS_Dio_00103] applies to the type Dio_ChannelType.♩(SRS_- SPAL_12263, SRS_Dio_12355)

### Dio_PortType

[SWS_Dio_00183] Definition of datatype Dio_PortType [

| Name | Dio_PortType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint |  |  |
| Range | 0..<number of ports> | – | Shall cover all available DIO Ports. |
| Description | Numeric ID of a DIO port. |  |  |
| Available via | Dio.h |  |  |

♩()

[SWS_Dio_00018] [Parameters of type Dio_PortType contain the numeric ID of a DIO port.♩()

[SWS_Dio_00181] [The mapping of ID is implementation specific but not config- urable.♩()

[SWS_Dio_00020] [For parameter values of type Dio_PortType, the user shall use the symbolic names provided by the configuration description.

Furthermore, [SWS_Dio_00103] applies to the type Dio_PortType.♩(SRS_SPAL_- 12263, SRS_Dio_12355)

### Dio_ChannelGroupType

[SWS_Dio_00184] Definition of datatype Dio_ChannelGroupType [

| Name | Dio_ChannelGroupType |  |
| --- | --- | --- |
| Kind | Structure |  |
| Elements | mask |  |
|  | Type | uint8/16/32 |
|  | Comment | This element mask which defines the positions of the channel group. |
|  | offset |  |
|  | Type | uint8 |
|  | Comment | This element shall be the position of the Channel Group on the port, counted from the LSB. |
|  | port |  |
|  | Type | Dio_PortType |
|  | Comment | This shall be the port on which the Channel group is defined. |
| Description | Type for the definition of a channel group, which consists of several adjoining channels within a port. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00021] [Dio_ChannelGroupType is the type for the definition of a chan- nel group, which consists of several adjoining channels within a port.♩(SRS_Dio_- 12004, SRS_Dio_12007)

[SWS_Dio_00022] [For parameter values of type Dio_ChannelGroupType, the user shall use the symbolic names provided by the configuration description.

Furthermore, [SWS_Dio_00056] applies to the type Dio_ChannelGroupType.♩

(SRS_SPAL_12263, SRS_Dio_12355)

### Dio_LevelType

[SWS_Dio_00185] Definition of datatype Dio_LevelType [

| Name | Dio_LevelType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint8 |  |  |
| Range | STD_LOW | 0x00 | Physical state 0V |
|  | STD_HIGH | 0x01 | Physical state 5V or 3.3V |
| Description | These are the possible levels a DIO channel can have (input or output) |  |  |
| Available via | Dio.h |  |  |

♩()

[SWS_Dio_00023] [Dio_LevelType is the type for the possible levels that a DIO channel can have (input or output).♩()

### Dio_PortLevelType

[SWS_Dio_00186] Definition of datatype Dio_PortLevelType [

| Name | Dio_PortLevelType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint |  |  |
| Range | 0...xxx | – | If the µC owns ports of different port widths (e.g. 4, 8,16...Bit) Dio_ PortLevelType inherits the size of the largest port |
| Description | If the µC owns ports of different port widths (e.g. 4, 8,16...Bit) Dio_PortLevelType inherits the size of the largest port. |  |  |
| Available via | Dio.h |  |  |

♩()

[SWS_Dio_00024] [Dio_PortLevelType is the type for the value of a DIO port. Furthermore, [SWS_Dio_00103] applies to the type Dio_PortLevelType.♩()

## Function definitions

This is a list of functions provided for upper layer modules.

### Dio_ReadChannel

[SWS_Dio_00133] Definition of API function Dio_ReadChannel [

| Service Name | Dio_ReadChannel |  |
| --- | --- | --- |
| Syntax | Dio_LevelType Dio_ReadChannel ( Dio_ChannelType ChannelId ) |  |
| Service ID [hex] | 0x00 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ChannelId | ID of DIO channel |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Dio_LevelType | STD_HIGH The physical level of the corresponding Pin is STD_HIGH STD_LOW The physical level of the corresponding Pin is STD_LOW |
| Description | Returns the value of the specified DIO channel. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00027] [The Dio_ReadChannel function shall return the value of the specified DIO channel.

Regarding the return value of the Dio_ReadChannel function, the requirements [SWS_Dio_00083] and [SWS_Dio_00084] are applicable.

Furthermore, the requirements [SWS_Dio_00005] and [SWS_Dio_00026] are applica- ble to the Dio_ReadChannel function.♩(SRS_Dio_12008)

[SWS_Dio_00074] [If development error detection is enabled, the services Dio_- ReadChannel, Dio_WriteChannel and Dio_FlipChannel shall check the "Chan- nelId" parameter to be valid within the current configuration. If the "ChannelId" param-

eter is invalid, the functions shall report the error code DIO_E_PARAM_INVALID_- CHANNEL_ID to the DET.♩(SRS_BSW_00323, SRS_SPAL_12448)

### Dio_WriteChannel

| Service Name | Dio_WriteChannel |
| --- | --- |
| Syntax | void Dio_WriteChannel ( Dio_ChannelType ChannelId, Dio_LevelType Level ) |
| Service ID [hex] | 0x01 |
| Sync/Async | Synchronous |
| Reentrancy | Reentrant |

[SWS_Dio_00134] Definition of API function Dio_WriteChannel [

q

Δ

| Parameters (in) | ChannelId | ID of DIO channel |
| --- | --- | --- |
|  | Level | Value to be written |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Service to set a level of a channel. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00028] [If the specified channel is configured as an output channel, the Dio_WriteChannel function shall set the specified Level for the specified channel.♩ (SRS_Dio_12005)

[SWS_Dio_00029] [If the specified channel is configured as an input channel, the Dio_WriteChannel function shall have no influence on the physical output.♩(SRS_- Dio_12005)

[SWS_Dio_00079] [If the specified channel is configured as an input channel, the Dio_WriteChannel function shall have no influence on the result of the next Read- Service.

Furthermore, the requirements [SWS_Dio_00005], [SWS_Dio_00119] and [SWS_Dio_00026] are applicable to the Dio_WriteChannel function.♩(SRS_- Dio_12005)

### Dio_ReadPort

[SWS_Dio_00135] Definition of API function Dio_ReadPort [

| Service Name | Dio_ReadPort |  |
| --- | --- | --- |
| Syntax | Dio_PortLevelType Dio_ReadPort ( Dio_PortType PortId ) |  |
| Service ID [hex] | 0x02 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | PortId | ID of DIO Port |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Dio_PortLevelType | Level of all channels of that port |
| Description | Returns the level of all channels of that port. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00031] [The Dio_ReadPort function shall return the level of all channels of that port.♩(SRS_Dio_12006)

[SWS_Dio_00104] [When reading a port which is smaller than the Dio_PortLevel- Type using the Dio_ReadPort function (see [SWS_Dio_00103]), the function shall set the bits corresponding to undefined port pins to 0.

Furthermore, the requirements [SWS_Dio_00005] and [SWS_Dio_00026] are applica- ble to the Dio_ReadPort function.♩(SRS_Dio_12006)

[SWS_Dio_00075] [If development error detection is enabled, the functions Dio_- ReadPort, Dio_WritePort and Dio_MaskedWritePort shall check the "PortId"

parameter to be valid within the current configuration. If the "PortId" parameter is in- valid, the functions shall report the error code DIO_E_PARAM_INVALID_PORT_ID to

the DET.♩(SRS_BSW_00323, SRS_SPAL_12448)

### Dio_WritePort

[SWS_Dio_00136] Definition of API function Dio_WritePort [

| Service Name | Dio_WritePort |  |
| --- | --- | --- |
| Syntax | void Dio_WritePort ( Dio_PortType PortId, Dio_PortLevelType Level ) |  |
| Service ID [hex] | 0x03 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | PortId | ID of DIO Port |
|  | Level | Value to be written |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Service to set a value of the port. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00034] [The Dio_WritePort function shall set the specified value for the specified port.♩(SRS_Dio_12003)

[SWS_Dio_00035] [When the Dio_WritePort function is called, DIO Channels that are configured as input shall remain unchanged.♩(SRS_Dio_12003)

[SWS_Dio_00105] [When writing a port which is smaller than the Dio_PortLevel- Type using the Dio_WritePort function (see [SWS_Dio_00103]), the function shall ignore the MSB.♩(SRS_Dio_12003)

[SWS_Dio_00108] [The Dio_WritePort function shall have no effect on channels within this port which are configured as input channels.

Furthermore, the requirements [SWS_Dio_00005], [SWS_Dio_00119] and [SWS_Dio_00026] are applicable to the Dio_WritePort function.♩(SRS_Dio_- 12003)

### Dio_ReadChannelGroup

[SWS_Dio_00137] Definition of API function Dio_ReadChannelGroup [

| Service Name | Dio_ReadChannelGroup |  |
| --- | --- | --- |
| Syntax | Dio_PortLevelType Dio_ReadChannelGroup ( const Dio_ChannelGroupType* ChannelGroupIdPtr ) |  |
| Service ID [hex] | 0x04 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ChannelGroupIdPtr | Pointer to ChannelGroup |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Dio_PortLevelType | Level of a subset of the adjoining bits of a port |
| Description | This Service reads a subset of the adjoining bits of a port. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00037] [The Dio_ReadChannelGroup function shall read a subset of the adjoining bits of a port (channel group).♩(SRS_Dio_12007)

[SWS_Dio_00092] [The Dio_ReadChannelGroup function shall do the masking of the channel group.♩(SRS_Dio_12007)

[SWS_Dio_00093] [The Dio_ReadChannelGroup function shall do the shifting so that the values read by the function are aligned to the LSB.

Furthermore, the requirements [SWS_Dio_00005], [SWS_Dio_00056], [SWS_Dio_00083],  [SWS_Dio_00084] and [SWS_Dio_00026] are applicable to

the Dio_ReadChannelGroup function.♩(SRS_Dio_12007)

[SWS_Dio_00114] [If development error detection is enabled, the functions Dio_- ReadChannelGroup and Dio_WriteChannelGroup shall check the "Channel GroupIdPtr" parameter to be valid within the current configuration. If the "Channel

GroupIdPtr" parameter is invalid, the functions shall report the error code DIO_E_- PARAM_INVALID_GROUP to the DET.♩(SRS_BSW_00323, SRS_SPAL_12448)

### Dio_WriteChannelGroup

| Service Name | Dio_WriteChannelGroup |
| --- | --- |
| Syntax | void Dio_WriteChannelGroup ( const Dio_ChannelGroupType* ChannelGroupIdPtr, Dio_PortLevelType Level ) |

[SWS_Dio_00138] Definition of API function Dio_WriteChannelGroup [

q

Δ

| Service ID [hex] | 0x05 |  |
| --- | --- | --- |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ChannelGroupIdPtr | Pointer to ChannelGroup |
|  | Level | Value to be written |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Service to set a subset of the adjoining bits of a port to a specified level. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00039] [The Dio_WriteChannelGroup function shall set a subset of the adjoining bits of a port (channel group) to a specified level.♩(SRS_Dio_12004)

[SWS_Dio_00040] [The Dio_WriteChannelGroup shall not change the remaining channels of the port and channels which are configured as input.♩(SRS_Dio_12004)

[SWS_Dio_00090] [The Dio_WriteChannelGroup function shall do the masking of the channel group.♩(SRS_Dio_12004)

[SWS_Dio_00091] [The function Dio_WriteChannelGroup shall do the shifting so that the values written by the function are aligned to the LSB.

Furthermore,	the	requirements	[SWS_Dio_00005],	[SWS_Dio_00056],

[SWS_Dio_00119] and [SWS_Dio_00026] are applicable for the Dio_WriteChan- nelGroup function.♩(SRS_Dio_12004)

### Dio_GetVersionInfo

[SWS_Dio_00139] Definition of API function Dio_GetVersionInfo [

| Service Name | Dio_GetVersionInfo |  |
| --- | --- | --- |
| Syntax | void Dio_GetVersionInfo ( Std_VersionInfoType* VersionInfo ) |  |
| Service ID [hex] | 0x12 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | None |  |
| Parameters (inout) | None |  |
| Parameters (out) | VersionInfo | Pointer to where to store the version information of this module. |
| Return value | None |  |
| Description | Service to get the version information of this module. |  |
| Available via | Dio.h |  |

♩(SRS_BSW_00411)

[SWS_Dio_00189] [If DET is enabled for the DIO Driver module, the function Dio_- GetVersionInfo shall raise DIO_E_PARAM_POINTER, if the argument is NULL pointer and return without any action.♩(SRS_BSW_00323)

### Dio_FlipChannel

[SWS_Dio_00190] Definition of API function Dio_FlipChannel [

| Service Name | Dio_FlipChannel |  |
| --- | --- | --- |
| Syntax | Dio_LevelType Dio_FlipChannel ( Dio_ChannelType ChannelId ) |  |
| Service ID [hex] | 0x11 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | ChannelId | ID of DIO channel |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | Dio_LevelType | STD_HIGH: The physical level of the corresponding Pin is STD_HIGH. STD_LOW: The physical level of the corresponding Pin is STD_LOW. |
| Description | Service to flip (change from 1 to 0 or from 0 to 1) the level of a channel and return the level of the channel after flip. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00191] [If the specified channel is configured as an output chan- nel, the Dio_FlipChannel function shall read level of the channel (requirements [SWS_Dio_00083] and [SWS_Dio_00084] are applicable) and invert it, then write the

inverted level to the channel. The return value shall be the inverted level of the specified channel.♩(SRS_Dio_12900)

[SWS_Dio_00192] [If the specified channel is configured as an input channel, the

Dio_FlipChannel function shall have no influence on the physical output.

The return value shall be the level of the specified channel.♩(SRS_Dio_12900) [SWS_Dio_00193] [If the specified channel is configured as an input channel, the Dio_FlipChannel function shall have no influence on the result of the next Read-

Service.

Furthermore, the requirements [SWS_Dio_00005], [SWS_Dio_00119] and [SWS_Dio_00026] are applicable to the Dio_FlipChannel function.♩(SRS_- Dio_12900)

### Dio_MaskedWritePort

[SWS_Dio_00300] Definition of API function Dio_MaskedWritePort [

| Service Name | Dio_MaskedWritePort |  |
| --- | --- | --- |
| Syntax | void Dio_MaskedWritePort ( Dio_PortType PortId, Dio_PortLevelType Level, Dio_PortLevelType Mask ) |  |
| Service ID [hex] | 0x13 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | PortId | ID of DIO Port |
|  | Level | Value to be written |
|  | Mask | Channels to be masked in the port |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Service to set the value of a given port with required mask. |  |
| Available via | Dio.h |  |

♩()

[SWS_Dio_00202] [The Dio_MaskedWritePort function shall set the specified value for the channels in the specified port if the corresponding bit in Mask is ’1’.♩ (SRS_Dio_12003)

[SWS_Dio_00203] [When the Dio_MaskedWritePort function is called, DIO Chan- nels that are configured as input shall remain unchanged.♩(SRS_Dio_12003)

[SWS_Dio_00204] [When writing a port which is smaller than the Dio_PortLevel- Type using the Dio_MaskedWritePort function (see [SWS_Dio_00103]), the func- tion shall ignore the MSB.♩(SRS_Dio_12003)

## Callback notifications

This chapter lists all functions provided by the Dio module to lower layers.

The Dio module does not provide any callback notifications. Callbacks related to the functionality of the Dio module are implemented in another module (ICU Driver and / or complex drivers).

## Scheduled functions

This chapter lists all functions called directly by the Basic Software Module Scheduler. The Dio module has no scheduled functions.

## Expected interfaces

This chapter lists all functions the Dio module requires from other modules.

### Mandatory Interfaces

None

### Optional Interfaces

This chapter defines all interfaces which are required to fulfill an optional functionality of the module.

### [SWS_Dio_00140] Definition of optional interfaces in module Dio [

| API Function | Header File | Description |
| --- | --- | --- |
| Det_ReportError | Det.h | Service to report development errors. |

♩()

# Sequence diagrams

The diagrams below show the sequences when calling the Dio_ReadChannel and Dio_WriteChannel service, they show normal operation mode. Since all other ser- vices which are defined in Chapter 8.3 have exactly the same synchronous behavior concerning, there are intentionally no further sequence diagrams in this document.

## Read a value from a digital I/O

Dio_ReadChannel(Dio_LevelType, Dio_ChannelType)

Request()

Level()

Dio_ReadChannel=STD_HIGH or STD_LOW()

«Peripheral» Dio Hardware

«module» Dio

User

Figure 9.1: Read Service Sequence Chart (normal operation mode)

## Write a value to a digital I/O

Dio_WriteChannel(Dio_ChannelType, Dio_LevelType)

Level()

Dio_WriteChannel()

«Peripheral» Dio Hardware

«module» Dio

User

Figure 9.2: Write Service Sequence Chart (normal operation mode)

# Configuration specification

This chapter defines configuration parameters and their clustering into containers.

## Containers and configuration parameters

The following chapters summarize all configuration parameters.

[SWS_Dio_00205] [The DIO module shall reject configurations with partition map- pings which are not supported by the implementation.♩()

### Variants

[SWS_Dio_00129] [At least one of the following variants has to be supported by im- plementation: VARIANT-PRE-COMPILE, VARIANT-LINK-TIME.♩(SRS_BSW_00396)

defaultValue = false

DioFlipChannelApi:  EcucBooleanParamDef

DioMaskedWritePortApi:  EcucBooleanParamDef

defaultValue = false

DioEcucPartitionRef:  EcucReferenceDef

EcucPartition:  EcucParamConfContainerDef

Dio: EcucModuleDef

lowerMultiplicity = 0 upperMultiplicity = *

+parameter

DioVersionInfoApi:  EcucBooleanParamDef

defaultValue = false

+parameter

upperMultiplicity = 1 lowerMultiplicity = 1

upperMultiplicity = 1 lowerMultiplicity = 0

DioDevErrorDetect:  EcucBooleanParamDef

DioGeneral:  EcucParamConfContainerDef

+subContainer

lowerMultiplicity = 0 upperMultiplicity = *

+container

+destination

+reference

+parameter

+container

+parameter

upperMultiplicity = * lowerMultiplicity = 0

DioChannelGroup:  EcucParamConfContainerDef

upperMultiplicity = * lowerMultiplicity = 0

upperMultiplicity = * lowerMultiplicity = 1

DioChannel:  EcucParamConfContainerDef

DioPort:  EcucParamConfContainerDef

DioConfig:  EcucParamConfContainerDef

+subContainer

+subContainer

Figure 10.1: DIO Module Configuration

### Dio

| SWS Item | [ECUC_Dio_00154] |
| --- | --- |
| Module Name | Dio |
| Description | Configuration of the Dio (Digital IO) module. |
| Post-Build Variant Support | false |
| Supported Config Variants | VARIANT-LINK-TIME, VARIANT-PRE-COMPILE |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| DioConfig | 1 | This container contains the configuration parameters and sub containers of the AUTOSAR DIO module. |
| DioGeneral | 1 | General DIO module configuration parameters. |

### DioGeneral

| SWS Item | [ECUC_Dio_00141] |
| --- | --- |
| Container Name | DioGeneral |
| Parent Container | Dio |
| Description | General DIO module configuration parameters. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Dio_00142] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioDevErrorDetect |  |  |
| Parent Container | DioGeneral |  |  |
| Description | Switches the development error detection and notification on or off. - true: detection and notification is enabled. - false: detection and notification is disabled. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00153] |
| --- | --- |
| Parameter Name | DioFlipChannelApi |
| Parent Container | DioGeneral |
| Description | Adds / removes the service Dio_FlipChannel() from the code. |
| Multiplicity | 1 |
| Type | EcucBooleanParamDef |
| Default value | – |
| Post-Build Variant Value | false |

q

Δ

| Value Configuration Class | Pre-compile time | X | All Variants |
| --- | --- | --- | --- |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00155] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioMaskedWritePortApi |  |  |
| Parent Container | DioGeneral |  |  |
| Description | Adds / removes the service Dio_MaskedWritePort() from the code. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00143] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioVersionInfoApi |  |  |
| Parent Container | DioGeneral |  |  |
| Description | Adds / removes the service Dio_ GetVersionInfo() from the code. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00156] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioEcucPartitionRef |  |  |
| Parent Container | DioGeneral |  |  |
| Description | Maps the DIO driver to zero or multiple ECUC partitions to make the modules API available in this partition. |  |  |
| Multiplicity | 0..* |  |  |
| Type | Reference to EcucPartition |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

| +parameter |  |  |
| --- | --- | --- |
| DioChannelGroupIdentification: EcucStringParamDef |  |  |
| symbolicNameValue = true |  |  |

+container

+subContainer

+parameter

+parameter

+destination

lowerMultiplicity = 0 upperMultiplicity = *

lowerMultiplicity = 0 upperMultiplicity = *

min = 0

max = 31

EcucReferenceDef

EcucPartition: EcucParamConfContainerDef

DioChannelGroupEcucPartitionRef:

DioPortOffset: EcucIntegerParamDef

+destination

+reference

+destination

lowerMultiplicity = 0 upperMultiplicity = *

min = 0

symbolicNameValue = true max = 4294967295

min = 0

max = 4294967295

DioChannelEcucPartitionRef:  EcucReferenceDef

DioChannelId:  EcucIntegerParamDef

DioPortMask:  EcucIntegerParamDef

+parameter

+parameter

+reference

lowerMultiplicity = 0 upperMultiplicity = *

upperMultiplicity = * lowerMultiplicity = 0

min = 0

symbolicNameValue = true max = 4294967295

upperMultiplicity = * lowerMultiplicity = 0

DioPortEcucPartitionRef:  EcucReferenceDef

DioChannel: EcucParamConfContainerDef

DioChannelGroup:  EcucParamConfContainerDef

DioPortId:  EcucIntegerParamDef

+reference

+subContainer

+subContainer

upperMultiplicity = * lowerMultiplicity = 1

DioPort: EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

DioConfig:  EcucParamConfContainerDef

Dio: EcucModuleDef

Figure 10.2: DioPort Configuration Container

### DioPort

| SWS Item | [ECUC_Dio_00144] |
| --- | --- |
| Container Name | DioPort |
| Parent Container | DioConfig |
| Description | Configuration of individual DIO ports, consisting of channels and possible channel groups. Note that this container definition does not explicitly define a symbolic name parameter. Instead, the container’s short name will be used in the Ecu Configuration Description to specify the symbolic name of the port. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Dio_00145] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioPortId |  |  |
| Parent Container | DioPort |  |  |
| Description | Numeric identifier of the DIO port. Not all MCU ports may be used for DIO, thus there may be "gaps" in the list of all IDs. This value will be assigned to the DIO port symbolic name (i.e. the SHORT-NAME of the DioPort container). |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range | 0 .. 4294967295 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |

q

Δ

|  | Link time | – |  |
| --- | --- | --- | --- |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

| SWS Item | [ECUC_Dio_00157] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioPortEcucPartitionRef |  |  |
| Parent Container | DioPort |  |  |
| Description | Maps the DIO ports to zero or multiple ECUC partitions. The ECUC partitions referenced are a subset of the ECUC partitions where the DIO driver is mapped to. |  |  |
| Multiplicity | 0..* |  |  |
| Type | Reference to EcucPartition |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| DioChannel | 0..* | Configuration of an individual DIO channel. |
| DioChannelGroup | 0..* | Definition and configuration of DIO channel groups. A channel group represents several adjoining DIO channels represented by a logical group. Note that this container definition does not explicitly define a symbolic name parameter. Instead, the container’s short name will be used in the Ecu Configuration Description to specify the symbolic name of the channel group. |

[SWS_Dio_00206] [The ECUC partitions referenced by DioPortEcucPartition- Ref shall be a subset of the ECUC partitions referenced by DioEcucPartitionRef.♩ ()

[SWS_Dio_00209] [If DioEcucPartitionRef references one or more ECUC parti- tions, DioPortEcucPartitionRef shall have a multiplicity of greater than zero and reference one or several of these ECUC partitions as well.♩()

### DioChannel

| SWS Item | [ECUC_Dio_00146] |
| --- | --- |
| Container Name | DioChannel |
| Parent Container | DioPort |
| Description | Configuration of an individual DIO channel. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Dio_00147] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioChannelId |  |  |
| Parent Container | DioChannel |  |  |
| Description | Channel Id of the DIO channel. This value will be assigned to the symbolic names. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range | 0 .. 4294967295 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

| SWS Item | [ECUC_Dio_00158] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioChannelEcucPartitionRef |  |  |
| Parent Container | DioChannel |  |  |
| Description | Maps a DIO channel to zero or multiple ECUC partitions. The ECUC partitions referenced are a subset of the ECUC partitions where the related DIO port is mapped to. |  |  |
| Multiplicity | 0..* |  |  |
| Type | Reference to EcucPartition |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

[SWS_Dio_00207] [The ECUC partitions referenced by DioChannelEcucParti- tionRef shall be a subset of the ECUC partitions referenced by DioPortEcucPar- titionRef.♩()

[SWS_Dio_00210] [If DioEcucPartitionRef references one or more ECUC parti- tions, DioChannelEcucPartitionRef shall have a multiplicity of greater than zero and reference one or several of these ECUC partitions as well.♩()

### DioChannelGroup

| SWS Item | [ECUC_Dio_00148] |
| --- | --- |
| Container Name | DioChannelGroup |
| Parent Container | DioPort |
| Description | Definition and configuration of DIO channel groups. A channel group represents several adjoining DIO channels represented by a logical group. Note that this container definition does not explicitly define a symbolic name parameter. Instead, the container’s short name will be used in the Ecu Configuration Description to specify the symbolic name of the channel group. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Dio_00149] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioChannelGroupIdentification |  |  |
| Parent Container | DioChannelGroup |  |  |
| Description | The DIO channel group is identified in DIO API by a pointer to a data structure (of type Dio_ChannelGroupType). That data structure contains the channel group information. This parameter contains the code fragment that has to be inserted in the API call of the calling module to get the address of the variable in memory which holds the channel group information. Example values are "&MyDioGroup1" or "&MyDioGroupArray[0]" |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucStringParamDef (Symbolic Name generated for this parameter) |  |  |
| Default value | – |  |  |
| Regular Expression | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

| SWS Item | [ECUC_Dio_00150] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioPortMask |  |  |
| Parent Container | DioChannelGroup |  |  |
| Description | This shall be the mask which defines the positions of the channel group. The channels shall consist of adjoining bits in the same port. The data type depends on the port width. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range | 0 .. 4294967295 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | X | VARIANT-LINK-TIME |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00151] |
| --- | --- |
| Parameter Name | DioPortOffset |
| Parent Container | DioChannelGroup |

q

Δ

| Description | The position of the Channel Group on the port, counted from the LSB. This value can be derived from DioPortMask. calculationFormula = Position of the first bit of DioPortMask which is set to ’1’ counted from LSB |  |  |
| --- | --- | --- | --- |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range | 0 .. 31 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | X | VARIANT-LINK-TIME |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Dio_00159] |  |  |
| --- | --- | --- | --- |
| Parameter Name | DioChannelGroupEcucPartitionRef |  |  |
| Parent Container | DioChannelGroup |  |  |
| Description | Maps a DIO channel group to zero or multiple ECUC partitions. The ECUC partitions referenced are a subset of the ECUC partitions where the related DIO port is mapped to. |  |  |
| Multiplicity | 0..* |  |  |
| Type | Reference to EcucPartition |  |  |
| Post-Build Variant Multiplicity | false |  |  |
| Post-Build Variant Value | false |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

[SWS_Dio_00208] [The ECUC partitions referenced by DioChannelGroupEcuc- PartitionRef shall be a subset of the ECUC partitions referenced by DioPortE- cucPartitionRef.♩()

[SWS_Dio_00211] [If DioPortEcucPartitionRef references one or more ECUC partitions, DioChannelGroupEcucPartitionRef shall have a multiplicity of greater than zero and reference one or several of these ECUC partitions as well.♩()

### DioConfig

| SWS Item | [ECUC_Dio_00152] |
| --- | --- |
| Container Name | DioConfig |
| Parent Container | Dio |
| Description | This container contains the configuration parameters and sub containers of the AUTOSAR DIO module. |
| Configuration Parameters |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| DioPort | 1..* | Configuration of individual DIO ports, consisting of channels and possible channel groups. Note that this container definition does not explicitly define a symbolic name parameter. Instead, the container’s short name will be used in the Ecu Configuration Description to specify the symbolic name of the port. |

## Published Information

For details refer to the chapter 10.3 "Published Information" in SWS_BSWGeneral.

# A  Not applicable requirements

[SWS_Dio_NA_00195] [These requirements are not applicable to this specifica- tion.♩(SRS_BSW_00101, SRS_BSW_00005, SRS_BSW_00006, SRS_BSW_00007, SRS_BSW_00009, SRS_BSW_00010, SRS_BSW_00160, SRS_BSW_00161, SRS_-

| BSW_00162, | SRS_BSW_00164, | SRS_BSW_00167, | SRS_BSW_00168, | SRS_- |
| --- | --- | --- | --- | --- |
| BSW_00170, | SRS_BSW_00172, | SRS_BSW_00304, | SRS_BSW_00306, | SRS_- |
| BSW_00307, | SRS_BSW_00308, | SRS_BSW_00309, | SRS_BSW_00314, | SRS_- |
| BSW_00321, | SRS_BSW_00325, | SRS_BSW_00328, | SRS_BSW_00330, | SRS_- |
| BSW_00331, | SRS_BSW_00333, | SRS_BSW_00334, | SRS_BSW_00335, | SRS_- |
| BSW_00336, | SRS_BSW_00339, | SRS_BSW_00341, | SRS_BSW_00342, | SRS_- |
| BSW_00343, | SRS_BSW_00347, | SRS_BSW_00357, | SRS_BSW_00359, | SRS_- |
| BSW_00360, | SRS_BSW_00369, | SRS_BSW_00373, | SRS_BSW_00375, | SRS_- |
| BSW_00377, | SRS_BSW_00378, | SRS_BSW_00384, | SRS_BSW_00399, | SRS_- |
| BSW_00400, | SRS_BSW_00404, | SRS_BSW_00405, | SRS_BSW_00406, | SRS_- |
| BSW_00413, | SRS_BSW_00416, | SRS_BSW_00417, | SRS_BSW_00422, | SRS_- |
| BSW_00423, | SRS_BSW_00424, | SRS_BSW_00425, | SRS_BSW_00426, | SRS_- |
| BSW_00427, | SRS_BSW_00428, | SRS_BSW_00429, | SRS_BSW_00432, | SRS_- |
| BSW_00433, | SRS_SPAL_00157, | SRS_SPAL_12057, | SRS_SPAL_12063, | SRS_- |
| SPAL_12067, SRS_SPAL_12068, SRS_SPAL_12069, SRS_SPAL_12075, |  |  |  | SRS_- |
| SPAL_12077, SRS_SPAL_12078, SRS_SPAL_12092, SRS_SPAL_12125, |  |  |  | SRS_- |
| SPAL_12129, SRS_SPAL_12163, SRS_SPAL_12169, SRS_SPAL_12265, SPAL_12267) |  |  |  | SRS_- |