| Document Title | Specification of Port Driver |
| --- | --- |
| Document Owner | AUTOSAR |
| Document Responsibility | AUTOSAR |
| Document Identification No | 40 |

| Document Status | published |
| --- | --- |
| Part of AUTOSAR Standard | Classic Platform |
| Part of Standard Release | R22-11 |

| Document Change History |  |  |  |
| --- | --- | --- | --- |
| Date | Release | Changed by | Description |
| 2022-11-24 | R22-11 | AUTOSAR Release Management | - Proper implementation of TPS_STDT_00042 |
| 2021-11-25 | R21-11 | AUTOSAR Release Management | - Removed [SWS_Port_00072] and [SWS_Port_00073] and corresponding notes |
| 2020-11-30 | R20-11 | AUTOSAR Release Management | - Added Enum values for Port_PinDirectionType - Editorial changes |
| 2019-11-28 | R19-11 | AUTOSAR Release Management | - MCAL Multicore Distribution - Changed Document Status from Final to published |
| 2018-10-31 | 4.4.0 | AUTOSAR Release Management | - MCAL Multicore Distribution (Draft) |
| 2017-12-08 | 4.3.1 | AUTOSAR Release Management | - minor corrections / clarifications / editorial changes; For details please refer to the ChangeDocumentation |
| 2016-11-30 | 4.3.0 | AUTOSAR Release Management | - Removed remaining references to DEM - Removed section "Variants" |

| 2015-07-31 | 4.2.2 | AUTOSAR Release Management | - Rephrased [SWS_Port_00077], [SWS_Port_00087], [SWS_Port_00087], [SWS_Port_00223] - Editorial changes on Chapter 7 - Remove [SWS_Port_0105] - Replace PORT_E_PARAM_CONFIG by PORT_E_INIT_FAILED |
| --- | --- | --- | --- |
| 2014-10-31 | 4.2.1 | AUTOSAR Release Management | - Editorial changes |
| 2013-10-31 | 4.1.2 | AUTOSAR Release Management | - Editorial changes - Removed chapter(s) on change documentation |
| 2013-03-15 | 4.1.1 | AUTOSAR Administration | - PORT_E_UNINIT removed from Port_GetVersionInfo API - Header file structure: MemMap.h changed to Port_MemMap.h and EcuM.h removed - Changed scope of configuration parameters from "module" to "local" - Removal of specification items covered by the new SWS BSW General - Formal rework |
| 2011-12-22 | 4.0.3 | AUTOSAR Administration | - Removed Port132 and updated Figure 1 - Rephrased [SWS_Port_00114] and [SWS_Port_00075] - Removed Port210 - Added Chapter 12 |
| 2010-09-30 | 3.1.5 | AUTOSAR Administration | - A lot of requirements are changed to make them atomic or /and more clear. No technical content changes. Look Chapter 14 for the complete list. - Debugging Concept Inserted. - Insertion of the new configuration parameter to enable or disable the mode of the port pins at run time. - Checking of Port_GetVersionInfo. - Legal disclaimer revised |

| 2009-02-04 | 3.1.2 | AUTOSAR CM | - Updated document to new SWS macros and updated links to generated content |
| --- | --- | --- | --- |
| 2009-07-24 | 3.1.3 | AUTOSAR Administration | - Legal disclaimer revised |
| 2008-08-13 | 3.1.1 | AUTOSAR Administration | - Table formatting corrected |
| 2008-02-01 | 3.0.2 | AUTOSAR Administration | - Update to Chapter 10 configuration. - Inclusion of Port Container - Inclusion of new SRS general requirements - Removal of redundant function: Dem_ReportErrorEvent() - Development errors and error codes added - Rewording of requirements (as part of the SWS Improvements) - Renaming of configuration parameter (PORT_PIN_DIRECTION_CHANG ES_ALLOWED -> PORT_SEP_PIN_DIRECTION_API) - Technical Office Improvements: wording improvements, alignment of API description. - Document meta information extended - Small layout adaptations made |

| 2007-01-24 | 2.1.15 | AUTOSAR Administration | - New API introduced: Port_SetPinMode - New Module type definition: Port_PinModeType - Updated to section 5.1.2: Inclusion of new file structure information - Inclusion of new pre-processor switch PortSetPinModeApi - New configurable parameter introduced: PortPinInitialMode - Rewording of requirement [SWS_Port_00105]. - Removal of redundant requirements PORT119 and PORT026 in requirements matrix. - Legal disclaimer revised - Release Notes added - "Advice for users" revised - "Revision Information" added |
| --- | --- | --- | --- |
| 2006-05-16 | 2.0 | AUTOSAR Administration | - Document structure adapted to common Release 2.0 SWS Template. - Major changes in chapter 10 - Structure of document changed partly - Other changes see chapter 11 |
| 2005-05-31 | 1.0 | AUTOSAR Administration | - Initial Release |

### Disclaimer

This work (specification and/or software implementation) and the material contained in it, as released by AUTOSAR, is for the purpose of information only. AUTOSAR and the companies that have contributed to it shall not be liable for any use of the work.

The material contained in this work is protected by copyright and other types of intel- lectual property rights. The commercial exploitation of the material contained in this work requires a license to such intellectual property rights.

This work may be utilized or reproduced without any modification, in any form or by any means, for informational purposes only. For any other purpose, no part of the work may be utilized or reproduced, in any form or by any means, without permission in writing from the publisher.

The work has been developed for automotive applications only. It has neither been developed, nor tested for non-automotive applications.

The word AUTOSAR and the AUTOSAR logo are registered trademarks.

# Contents

- Introduction and functional overview	8

- Acronyms and abbreviations	10

- Related documentation	11

  - Input documents	11

  - Related standards and norms	11

  - Related specification	11

- Constraints and assumptions	12

  - Limitations	12

  - Applicability to car domains	12

- Dependencies to other modules	13

  - File structure	13

    - Code file structure	13

- Requirements traceability	14

- Functional specification	20

  - General Behaviour	20

    - Background & Rationale	20

    - Requirements	20

      - Configuration of Port Pin Properties	20

      - Switch port pin direction	21

      - Refresh port direction	21

      - Configuration of unused Ports and Port Pins	21

      - Configuration of symbolic names	21

      - Atomicity of port access	21

    - Version Check	22

      - Background and Rationale	22

      - Requirements	22

  - Error classification	22

    - Runtime Errors	22

    - Transient Faults	23

    - Production Errors	23

    - Extended Production Errors	23

  - API Parameter checking	23

- API specification	24

  - Imported types	24

  - Type definitions	24

    - Port_ConfigType	24

    - Port_PinType	24

    - Port_PinDirectionType	25

    - Port_PinModeType	25

  - Function definitions	26

    - Port_Init	26

    - Port_SetPinDirection	28

    - Port_RefreshPortDirection	29

    - Port_GetVersionInfo	29

    - Port_SetPinMode	30

  - Call-back notifications	30

  - Scheduled functions	30

  - Expected Interfaces	31

    - Mandatory Interfaces	31

    - Optional Interfaces	31

    - Configurable Interfaces	31

- Sequence diagrams	32

  - Overall Configuration of Ports	32

  - Set the direction of a Port Pin	32

  - Refresh the direction of all Port Pins	33

  - Change the mode of a Port Pin	33

- Configuration specification	34

  - How to read this chapter	34

  - Containers and configuration parameters	34

    - Port	34

    - PortContainer	35

    - PortGeneral	36

    - PortPin	38

  - Constraints	42

  - Published Information	43

A  Not applicable requirements	44

# Introduction and functional overview

This specification specifies the functionality, API and the configuration of the AUTOSAR Basic Software module PORT Driver.

This driver specification is applicable for on-chip ports and port pins.

This module shall provide the service for initializing the whole PORT structure of the microcontroller. Many ports and port pins can be assigned to various functionalities, e.g.

- General purpose I/O

- ADC

- SPI

- SCI

- PWM

- CAN

- LIN

- etc

For this reason, there shall be an overall configuration and initialization of this port structure. The configuration and mode of these port pins is microcontroller and ECU dependent.

Port initialisation data shall be written to each port as efficiently as possible.

This PORT driver module shall complete the overall configuration and initialisation of the port structure which is used in the DIO driver module. Therefore, the DIO driver works on pins and ports which are configured by the PORT driver.

The PORT driver shall be initialised prior to use of the DIO functions. Otherwise DIO functions will exhibit undefined behaviour.

The diagram below identifies the PORT driver functions, and the structure of the PORT driver and DIO driver within the MCAL software layer [1].

| Driver: | Name for a Port Pin: | Name for Subset of Adjacent pins on one port: | Name for a whole port: |
| --- | --- | --- | --- |
| DIO Driver | Channel | Channel Group | Port |
| PORT Driver | Port pin | – | Port |

Figure 1.1

# Acronyms and abbreviations

The following table summarizes the expressions used within the PORT driver.

| Abbreviation / Acronym: | Description: |
| --- | --- |
| DEM | Diagnostic Event Manager [2] |
| DET | Default Error Tracer [3] |
| MCU | MicroController Unit |
| Port Pin | Represents a single configurable input or output pin on an MCU device. |
| Port | Represents a whole configurable port on an MCU device. |
| Physical Level (Input) | Two states are possible: LOW/HIGH |
| Physical Level (Output) | Two states are possible: LOW/HIGH |

Table 2.1: Acronyms and abbreviations

# Related documentation

## Input documents

- Layered Software Architecture AUTOSAR_EXP_LayeredSoftwareArchitecture

- Specification of Diagnostic Event Manager AUTOSAR_SWS_DiagnosticEventManager

- Specification of Default Error Tracer AUTOSAR_SWS_DefaultErrorTracer

- General Specification of Basic Software Modules AUTOSAR_SWS_BSWGeneral

- General Requirements on Basic Software Modules AUTOSAR_SRS_BSWGeneral

- General Requirements on SPAL AUTOSAR_SRS_SPALGeneral

- Requirements on Port Driver AUTOSAR_SRS_PortDriver

- Specification of MCU Driver AUTOSAR_SWS_MCUDriver

## Related standards and norms

1. EC 7498-1 The Basic Model, IEC Norm, 1994

## Related specification

AUTOSAR provides a General Specification on Basic Software modules [4, SWS BSW General], which is also valid for Port Driver.

Thus, the specification SWS BSW General shall be considered as additional and re- quired specification for Port Driver.

# Constraints and assumptions

## Limitations

Limitations for the PORT driver are specified as followed:

- It is the user’s responsibility to ensure that the same Port/Port pin is not being accessed in parallel by different entities in the same system, e.g. by two tasks configuring the same port or two tasks configuring the same pin, or two tasks configuring different pins on the same port.

## Applicability to car domains

No restrictions

# Dependencies to other modules

Other driver modules may be dependent on the PORT driver depending on the avail- able functionality of individual port pins on an MCU. For example, an MCU pin may be configurable as a DIO or SPI pin. Therefore, the DIO and/or the SPI driver modules may be dependent on the PORT module to configure the pin for the desired function- ality.

## File structure

### Code file structure

For details refer to the chapter 5.1.6 "Code file structure" in [4].

# Requirements traceability

This chapter refers to the input requirements specified in the SRS documents (Software Requirements Specifications [5], [6], [7]) that are applicable for this software module.

The table below lists the specification items of the PORT driver SWS document that satisfy the input requirements. Only functional requirements are referenced.

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_BSW_00005] | Modules of the µC Abstraction Layer (MCAL) may not have hard coded horizontal interfaces | [SWS_Port_NA_00227] |
| [SRS_BSW_00006] | The source code of software modules above the µC Abstraction Layer (MCAL) shall not be processor and compiler dependent. | [SWS_Port_NA_00227] |
| [SRS_BSW_00007] | All Basic SW Modules written in C language shall conform to the MISRA C 2012 Standard. | [SWS_Port_NA_00227] |
| [SRS_BSW_00010] | The memory consumption of all Basic SW Modules shall be documented for a defined configuration for all supported platforms. | [SWS_Port_NA_00227] |
| [SRS_BSW_00101] | The Basic Software Module shall be able to initialize variables and hardware in a separate initialization function | [SWS_Port_00001] [SWS_Port_00002] [SWS_Port_00041] [SWS_Port_00042] |
| [SRS_BSW_00159] | All modules of the AUTOSAR Basic Software shall support a tool based configuration | [SWS_Port_00004] |
| [SRS_BSW_00160] | Configuration files of AUTOSAR Basic SW module shall be readable for human beings | [SWS_Port_NA_00227] |
| [SRS_BSW_00161] | The AUTOSAR Basic Software shall provide a microcontroller abstraction layer which provides a standardized interface to higher software layers | [SWS_Port_NA_00227] |
| [SRS_BSW_00162] | The AUTOSAR Basic Software shall provide a hardware abstraction layer | [SWS_Port_NA_00227] |
| [SRS_BSW_00164] | The Implementation of interrupt service routines shall be done by the Operating System, complex drivers or modules | [SWS_Port_NA_00227] |
| [SRS_BSW_00167] | All AUTOSAR Basic Software Modules shall provide configuration rules and constraints to enable plausibility checks | [SWS_Port_NA_00227] |
| [SRS_BSW_00168] | SW components shall be tested by a function defined in a common API in the Basis-SW | [SWS_Port_NA_00227] |
| [SRS_BSW_00170] | The AUTOSAR SW Components shall provide information about their dependency from faults, signal qualities, driver demands | [SWS_Port_NA_00227] |

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_BSW_00172] | The scheduling strategy that is built inside the Basic Software Modules shall be compatible with the strategy used in the system | [SWS_Port_NA_00227] |
| [SRS_BSW_00307] | Global variables naming convention | [SWS_Port_NA_00227] |
| [SRS_BSW_00308] | AUTOSAR Basic Software Modules shall not define global data in their header files, but in the C file | [SWS_Port_NA_00227] |
| [SRS_BSW_00309] | All AUTOSAR Basic Software Modules shall indicate all global data with read-only purposes by explicitly assigning the const keyword | [SWS_Port_NA_00227] |
| [SRS_BSW_00321] | The version numbers of AUTOSAR Basic Software Modules shall be enumerated according specific rules | [SWS_Port_NA_00227] |
| [SRS_BSW_00323] | All AUTOSAR Basic Software Modules shall check passed API parameters for validity | [SWS_Port_00087] |
| [SRS_BSW_00325] | The runtime of interrupt service routines and functions that are running in interrupt context shall be kept short | [SWS_Port_NA_00227] |
| [SRS_BSW_00327] | Error values naming convention | [SWS_Port_00051] |
| [SRS_BSW_00328] | All AUTOSAR Basic Software Modules shall avoid the duplication of code | [SWS_Port_NA_00227] |
| [SRS_BSW_00330] | It shall be allowed to use macros instead of functions where source code is used and runtime is critical | [SWS_Port_NA_00227] |
| [SRS_BSW_00331] | All Basic Software Modules shall strictly separate error and status information | [SWS_Port_NA_00227] |
| [SRS_BSW_00333] | For each callback function it shall be specified if it is called from interrupt context or not | [SWS_Port_NA_00227] |
| [SRS_BSW_00334] | All Basic Software Modules shall provide an XML file that contains the meta data | [SWS_Port_NA_00227] |
| [SRS_BSW_00335] | Status values naming convention | [SWS_Port_NA_00227] |
| [SRS_BSW_00336] | Basic SW module shall be able to shutdown | [SWS_Port_NA_00227] |
| [SRS_BSW_00337] | Classification of development errors | [SWS_Port_00051] |
| [SRS_BSW_00341] | Module documentation shall contains all needed informations | [SWS_Port_NA_00227] |
| [SRS_BSW_00342] | It shall be possible to create an AUTOSAR ECU out of modules provided as source code and modules provided as object code, even mixed | [SWS_Port_NA_00227] |
| [SRS_BSW_00343] | The unit of time for specification and configuration of Basic SW modules shall be preferably in physical time unit | [SWS_Port_NA_00227] |
| [SRS_BSW_00344] | BSW Modules shall support link-time configuration | [SWS_Port_NA_00227] |

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_BSW_00347] | A Naming seperation of different instances of BSW drivers shall be in place | [SWS_Port_NA_00227] |
| [SRS_BSW_00357] | For success/failure of an API call a standard return type shall be defined | [SWS_Port_NA_00227] |
| [SRS_BSW_00358] | The return type of init() functions implemented by AUTOSAR Basic Software Modules shall be void | [SWS_Port_00140] |
| [SRS_BSW_00359] | All AUTOSAR Basic Software Modules callback functions shall avoid return types other than void if possible | [SWS_Port_NA_00227] |
| [SRS_BSW_00360] | AUTOSAR Basic Software Modules callback functions are allowed to have parameters | [SWS_Port_NA_00227] |
| [SRS_BSW_00371] | No description | [SWS_Port_NA_00227] |
| [SRS_BSW_00373] | The main processing function of each AUTOSAR Basic Software Module shall be named according the defined convention | [SWS_Port_NA_00227] |
| [SRS_BSW_00375] | Basic Software Modules shall report wake-up reasons | [SWS_Port_NA_00227] |
| [SRS_BSW_00377] | A Basic Software Module can return a module specific types | [SWS_Port_NA_00227] |
| [SRS_BSW_00378] | AUTOSAR shall provide a boolean type | [SWS_Port_NA_00227] |
| [SRS_BSW_00385] | List possible error notifications | [SWS_Port_00051] |
| [SRS_BSW_00395] | The Basic Software Module specifications shall list all configuration parameter dependencies | [SWS_Port_NA_00227] |
| [SRS_BSW_00398] | The link-time configuration is achieved on object code basis in the stage after compiling and before linking | [SWS_Port_NA_00227] |
| [SRS_BSW_00404] | BSW Modules shall support post-build configuration | [SWS_Port_00041] |
| [SRS_BSW_00406] | A static status variable denoting if a BSW module is initialized shall be initialized with value 0 before any APIs of the BSW module is called | [SWS_Port_00051] [SWS_Port_00087] |
| [SRS_BSW_00413] | An index-based accessing of the instances of BSW modules shall be done | [SWS_Port_NA_00227] |
| [SRS_BSW_00414] | Init functions shall have a pointer to a configuration structure as single parameter | [SWS_Port_00121] |
| [SRS_BSW_00416] | The sequence of modules to be initialized shall be configurable | [SWS_Port_NA_00227] |
| [SRS_BSW_00417] | Software which is not part of the SW-C shall report error events only after the Dem is fully operational. | [SWS_Port_NA_00227] |

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_BSW_00419] | If a pre-compile time configuration parameter is implemented as const it should be placed into a separate c-file | [SWS_Port_NA_00227] |
| [SRS_BSW_00423] | BSW modules with AUTOSAR interfaces shall be describable with the means of the SW-C Template | [SWS_Port_NA_00227] |
| [SRS_BSW_00424] | BSW module main processing functions shall not be allowed to enter a wait state | [SWS_Port_NA_00227] |
| [SRS_BSW_00425] | The BSW module description template shall provide means to model the defined trigger conditions of schedulable objects | [SWS_Port_NA_00227] |
| [SRS_BSW_00426] | BSW Modules shall ensure data consistency of data which is shared between BSW modules | [SWS_Port_NA_00227] |
| [SRS_BSW_00427] | ISR functions shall be defined and documented in the BSW module description template | [SWS_Port_NA_00227] |
| [SRS_BSW_00428] | A BSW module shall state if its main processing function(s) has to be executed in a specific order or sequence | [SWS_Port_NA_00227] |
| [SRS_BSW_00429] | Access to OS is restricted | [SWS_Port_NA_00227] |
| [SRS_BSW_00432] | Modules should have separate main processing functions for read/receive and write/transmit data path | [SWS_Port_NA_00227] |
| [SRS_BSW_00433] | Main processing functions are only allowed to be called from task bodies provided by the BSW Scheduler | [SWS_Port_NA_00227] |
| [SRS_BSW_00437] | Memory mapping shall provide the possibility to define RAM segments which are not to be initialized during startup | [SWS_Port_NA_00227] |
| [SRS_BSW_00439] | Enable BSW modules to handle interrupts | [SWS_Port_NA_00227] |
| [SRS_BSW_00440] | The callback function invocation by the BSW module shall follow the signature provided by RTE to invoke servers via Rte_Call API | [SWS_Port_NA_00227] |
| [SRS_Port_12001] | The Port driver shall allow the static configuration of the following options for each port | [SWS_Port_00004] [SWS_Port_00079] |
| [SRS_Port_12300] | Ports and port pins that are not used shall be set to a defined state | [SWS_Port_00005] |
| [SRS_Port_12302] | The port driver shall allow the static configuration of the port pin names | [SWS_Port_00006] |
| [SRS_Port_12405] | The Port driver shall provide a service for setting the direction of port pins during runtime | [SWS_Port_00063] [SWS_Port_00086] [SWS_Port_00138] |
| [SRS_Port_12406] | The Port driver shall provide a service to refresh the direction of all configured ports | [SWS_Port_00060] [SWS_Port_00061] |

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_SPAL_00157] | All drivers and handlers of the AUTOSAR Basic Software shall implement notification mechanisms of drivers and handlers | [SWS_Port_NA_00227] |
| [SRS_SPAL_12056] | All driver modules shall allow the static configuration of notification mechanism | [SWS_Port_NA_00227] |
| [SRS_SPAL_12057] | All driver modules shall implement an interface for initialization | [SWS_Port_00041] [SWS_Port_00042] [SWS_Port_00043] |
| [SRS_SPAL_12063] | All driver modules shall only support raw value mode | [SWS_Port_NA_00227] |
| [SRS_SPAL_12064] | All driver modules shall raise an error if the change of the operation mode leads to degradation of running operations | [SWS_Port_NA_00227] |
| [SRS_SPAL_12067] | All driver modules shall set their wake-up conditions depending on the selected operation mode | [SWS_Port_NA_00227] |
| [SRS_SPAL_12068] | The modules of the MCAL shall be initialized in a defined sequence | [SWS_Port_NA_00227] |
| [SRS_SPAL_12069] | All drivers of the SPAL that wake up from a wake-up interrupt shall report the wake-up reason | [SWS_Port_NA_00227] |
| [SRS_SPAL_12075] | All drivers with random streaming capabilities shall use application buffers | [SWS_Port_NA_00227] |
| [SRS_SPAL_12077] | All drivers shall provide a non blocking implementation | [SWS_Port_NA_00227] |
| [SRS_SPAL_12078] | The drivers shall be coded in a way that is most efficient in terms of memory and runtime resources | [SWS_Port_NA_00227] |
| [SRS_SPAL_12092] | The driver’s API shall be accessed by its handler or manager | [SWS_Port_NA_00227] |
| [SRS_SPAL_12125] | All driver modules shall only initialize the configured resources | [SWS_Port_00041] [SWS_Port_00042] |
| [SRS_SPAL_12129] | The ISRs shall be responsible for resetting the interrupt flags and calling the according notification function | [SWS_Port_NA_00227] |
| [SRS_SPAL_12163] | All driver modules shall implement an interface for de-initialization | [SWS_Port_00003] |
| [SRS_SPAL_12169] | All driver modules that provide different operation modes shall provide a service for mode selection | [SWS_Port_NA_00227] |
| [SRS_SPAL_12263] | The implementation of all driver modules shall allow the configuration of specific module parameter types at link time | [SWS_Port_00041] |
| [SRS_SPAL_12265] | Configuration data shall be kept constant | [SWS_Port_NA_00227] |
| [SRS_SPAL_12267] | Wakeup sources shall be initialized by MCAL drivers and/or the MCU driver | [SWS_Port_NA_00227] |
| [SRS_SPAL_12448] | All driver modules shall have a specific behavior after a development error detection | [SWS_Port_00077] |

q

Δ

| Requirement | Description | Satisfied by |
| --- | --- | --- |
| [SRS_SPAL_12461] | Specific rules regarding initialization of controller registers shall apply to all driver implementations | [SWS_Port_00113] [SWS_Port_00214] [SWS_Port_00215] [SWS_Port_00217] [SWS_Port_00218] |
| [SRS_SPAL_12462] | The register initialization settings shall be published | [SWS_Port_NA_00227] |
| [SRS_SPAL_12463] | The register initialization settings shall be combined and forwarded | [SWS_Port_NA_00227] |

Table 6.1: RequirementsTracing

# Functional specification

## General Behaviour

### Background & Rationale

[SWS_Port_00001] [The PORT Driver module shall initialize the whole port structure of the microcontroller.♩(SRS_BSW_00101)

Note: Defining the order in which the ports and port pins are configured is the task of the configuration tool.

### Requirements

### Configuration of Port Pin Properties

[SWS_Port_00004] [The PORT Driver module shall allow the configuration of different functionality for each port and port pin, e.g. ADC, SPI, DIO etc.♩(SRS_BSW_00159, SRS_Port_12001)

The configuration of the port (i.e. whole port or single port pin) is microcontroller de- pendent.

[SWS_Port_00079] [The PORT Driver module shall provide additional configurations for the MCU port/port pins:

        - Pin direction (input/output)

        - Pin level initial value

        - Pin direction changeable during runtime (yes/no).

        - Port mode changeable during runtime.

♩(SRS_Port_12001)

[SWS_Port_00081] [The PORT Driver module shall provide a number of optional con- figurations for the MCU ports and port pins (if supported by hardware):

        - Slew rate control

        - Activation of internal pull-ups

        - Input Thresholds

        - Pin driven mode (push-pull / open drain).

        - Type of Readback support (pin level, output register value).

♩()

[SWS_Port_00082] [The PORT Driver module shall not provide the facility to configure pin level inversion. The default value shall be set (i.e. not inverted).♩()

Note: The IO Hardware Abstraction layer shall carry out level inversion.

### Switch port pin direction

[SWS_Port_00137] [For the port pins configured as changeable using the configura- tion tool, the PORT driver shall allow the user to change the direction of port pins during runtime.♩()

[SWS_Port_00138] [If the MCU port control hardware provides an output latch for setting the output level on a port pin, switching the port pin direction shall not alter the level set in this output latch.♩(SRS_Port_12405)

### Refresh port direction

[SWS_Port_00066] [For refreshing of the port on the microcontroller, the PORT driver shall allow the user to refresh the direction of those port pins whose direction is set by configuration and cannot be changed dynamically.♩()

### Configuration of unused Ports and Port Pins

[SWS_Port_00005] [The PORT Driver module shall configure all ports and port pins that are not used (neither as GPIO nor special purpose IO) to be set to a defined state by the PORT Driver module configuration.♩(SRS_Port_12300)

### Configuration of symbolic names

[SWS_Port_00006] [The user of the PORT Driver module shall configure the symbolic names of the port pins of the MCU.♩(SRS_Port_12302)

[SWS_Port_00207] [These symbolic names for the individual port pins (e.g. PORT_ A_PIN_0) shall be defined in the configuration tool.♩()

[SWS_Port_00208] [The PORT Driver module’s implementer shall publish the sym- bolic names through the file Port.h♩()

### Atomicity of port access

[SWS_Port_00075] [The PORT Driver module shall provide atomic access to all ports and port pins.♩()

Note:

An atomic access is a non interruptible access to Microcontroller registers by the use of either atomic instructions or the usage of an exclusive area (interrupt disabling for example) provided by the basic software scheduler module.

### Version Check

### Background and Rationale

The integration of incompatible files shall be avoided. Minimum implementation is the version check of the header file inside the .c file (version numbers of .c and .h files shall be identical).

### Requirements

The Port module shall avoid the integration of incompatible files by the following pre- processor checks:

For details refer to the chapter 5.1.8 "Version Check" in [4, SWS BSW General].

## Error classification

### [SWS_Port_00051] [

| Type of error | Related error code | Error value |
| --- | --- | --- |
| Invalid Port Pin ID requested | PORT_E_PARAM_PIN | 0x0A |
| Port Pin not configured as changeable | PORT_E_DIRECTION_UNCHANGEABLE | 0x0B |
| API Port_Init service called with wrong parameter | PORT_E_INIT_FAILED | 0x0C |
| API Port_SetPinMode service called when mode is unchangeable. | PORT_E_PARAM_INVALID_MODE | 0x0D |
| API Port_SetPinMode service called when mode is unchangeable. | PORT_E_MODE_UNCHANGEABLE | 0x0E |
| API service called without module initialization | PORT_E_UNINIT | 0x0F |
| APIs called with a Null Pointer | PORT_E_PARAM_POINTER | 0x10 |

♩(SRS_BSW_00327, SRS_BSW_00337, SRS_BSW_00385, SRS_BSW_00406)

### Runtime Errors

There are no runtime errors.

### Transient Faults

There are no transcient faults.

### Production Errors

There are no production errors.

### Extended Production Errors

There are no extended production errors.

## API Parameter checking

[SWS_Port_00077] [If development error detection is enabled the Port Driver module shall check the function parameters in the order in which they are passed and skip further parameter checking if one check fails.

Example: For the function Port_SetPinDirection, the first parameter to be passed is the pin ID. This parameter shall identify the relevant port pin of the MCU’s port. The second parameter passed corresponds to the direction to change on the port pin.♩ (SRS_SPAL_12448)

[SWS_Port_00087] [If development error detection is enabled and the Port Driver module has detected an error, the desired functionality shall be skipped and the requested service shall return without any action.♩(SRS_BSW_00323, SRS_BSW_- 00406)

See table below for a list of the Det errors reported by each function.

| Function: | Error Condition: | Realted error value: |
| --- | --- | --- |
| Port_- SetPinDirection | Incorrect Port Pin ID passed | PORT_E_PARAM_PIN |
|  | Port Pin not configured as changeable | PORT_E_DIRECTION_UNCHANGEABLE |
| Port_Init | Port_Init service called with wrong parameter. | PORT_E_INIT_FAILED |
| Port_SetPinMode | Incorrect Port Pin ID passed | PORT_E_PARAM_PIN |
|  | Port Pin Mode passed not valid | PORT_E_PARAM_INVALID_MODE |
|  | Port_SetPinMode service called when the mode is unchangeable | PORT_E_MODE_UNCHANGEABLE |
| Port_- SetPinDirection, Port_SetPinMode Port_RefreshPort- Direction | API service called prior to module initialization | PORT_E_UNINIT |
| Port_- GetVersionInfo | Api called with a NULL Pointer Parameter | PORT_E_PARAM_POINTER |

# API specification

## Imported types

In this chapter, all types included from the following modules are listed:

### [SWS_Port_00129] [

| Module | Header File | Imported Type |
| --- | --- | --- |
| Std | Std_Types.h | Std_ReturnType |
|  | Std_Types.h | Std_VersionInfoType |

♩()

## Type definitions

### Port_ConfigType [SWS_Port_00228] [

| Name | Port_ConfigType |  |
| --- | --- | --- |
| Kind | Structure |  |
| Elements | Hardware Dependent Structure |  |
|  | Type | – |
|  | Comment | The contents of the initialization data structure are specific to the microcontroller. |
| Description | Type of the external data structure containing the initialization data for this module. |  |
| Available via | Port.h |  |

♩()

### Port_PinType [SWS_Port_00229] [

| Name | Port_PinType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint |  |  |
| Range | 0 - <number of port pins:> | – | Shall cover all available port pins. The type should be chosen for the specific MCU platform (best performance). |
| Description | Data type for the symbolic name of a port pin. |  |  |
| Available via | Port.h |  |  |

♩()

[SWS_Port_00013] [The type Port_PinType shall be used for the symbolic name of a Port Pin.♩()

[SWS_Port_00219] [The type Port_PinType shall be uint8, uint16 or uint32

based on the specific MCU platform.♩()

Note: The user shall use the symbolic names provided by the configuration tool.

### Port_PinDirectionType [SWS_Port_00230] [

| Name | Port_PinDirectionType |  |  |
| --- | --- | --- | --- |
| Kind | Enumeration |  |  |
| Range | PORT_PIN_IN | 0x00 | Sets port pin as input. |
|  | PORT_PIN_OUT | 0x01 | Sets port pin as output. |
| Description | Possible directions of a port pin. |  |  |
| Available via | Port.h |  |  |

♩()

[SWS_Port_00046] [The type Port_PinDirectionType is a type for defining the direction of a Port Pin.♩()

[SWS_Port_00220] [The type Port_PinDirectionType shall be of enumeration type having range as PORT_PIN_IN and PORT_PIN_OUT.♩()

### Port_PinModeType [SWS_Port_00231] [

| Name | Port_PinModeType |  |  |
| --- | --- | --- | --- |
| Kind | Type |  |  |
| Derived from | uint |  |  |
| Range | Implementation specific | – | As several port pin modes shall be configurable on one pin, the range shall be determined by the implementation. |
| Description | Different port pin modes. |  |  |
| Available via | Port.h |  |  |

♩()

[SWS_Port_00124] [A port pin shall be configurable with a number of port pin modes (type Port_PinModeType).♩()

[SWS_Port_00212] [The type Port_PinModeType shall be used with the function call Port_SetPinMode.♩()

[SWS_Port_00221] [The type Port_PinModeType shall be uint8, uint16 or

uint32.♩()

## Function definitions

This is a list of functions provided for upper layer modules.

### Port_Init [SWS_Port_00140] [

| Service Name | Port_Init |  |
| --- | --- | --- |
| Syntax | void Port_Init ( const Port_ConfigType* ConfigPtr ) |  |
| Service ID [hex] | 0x00 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Non Reentrant |  |
| Parameters (in) | ConfigPtr | Pointer to configuration set. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Initializes the Port Driver module. |  |
| Available via | Port.h |  |

♩(SRS_BSW_00358)

[SWS_Port_00041] [The function Port_Init shall initialize ALL ports and port pins with the configuration set pointed to by the parameter ConfigPtr.♩(SRS_BSW_- 00101, SRS_BSW_00404, SRS_SPAL_12263, SRS_SPAL_12057, SRS_SPAL_-

12125)

[SWS_Port_00078] [The Port Driver module’s environment shall call the function

Port_Init first in order to initialize the port for use.♩()

[SWS_Port_00213] [If Port_Init function is not called first, then no operation can occur on the MCU ports and port pins.♩()

[SWS_Port_00042] [The function Port_Init shall initialize all configured re- sources.♩(SRS_BSW_00101, SRS_SPAL_12057, SRS_SPAL_12125)

The function Port_Init shall apply the following rules regarding initialisation of con- troller registers.

- [SWS_Port_00113] [If the hardware allows for only one usage of the register, the driver module implementing that functionality is responsible for initializing the register.♩(SRS_SPAL_12461)

- [SWS_Port_00214] [If the register can affect several hardware modules and if it is an I/O register it shall be initialised by this PORT driver.♩(SRS_SPAL_12461)

- [SWS_Port_00215] [If the register can affect several hardware modules and if it is not an I/O register, it shall be initialised by the MCU driver [8].♩(SRS_SPAL_- 12461)

- [SWS_Port_00217] [One-time writable registers that require initialisation directly after reset shall be initialised by the startup code.♩(SRS_SPAL_12461)

- [SWS_Port_00218] [All the other registers not mentioned before, shall be ini- tialised by the start-up code.♩(SRS_SPAL_12461)

[SWS_Port_00043] [The function Port_Init shall avoid glitches and spikes on the affected port pins.♩(SRS_SPAL_12057)

[SWS_Port_00071] [The Port Driver module’s environment shall call the function

Port_Init after a reset in order to reconfigure the ports and port pins of the MCU.♩()

[SWS_Port_00002] [The function Port_Init shall initialize all variables used by the PORT driver module to an initial state.♩(SRS_BSW_00101)

[SWS_Port_00003] [The Port Driver module’s environment may also uses the function Port_Init to initialize the driver software and reinitialize the ports and port pins to another configured state depending on the configuration set passed to this function.♩ (SRS_SPAL_12163)

Note: In some cases, MCU port control hardware provides an output latch for setting the output level on a port pin that may be used as a DIO port pin.

[SWS_Port_00055] [The function Port_Init shall set the port pin output latch to a default level (defined during configuration) before setting the port pin direction to output.♩()

Requirement [SWS_Port_00055] ensures that the default level is immediately output on the port pin when it is set to an output port pin.

Example: On some MCU’s, after a power-on-reset, a DIO configurable port pin will be configured as an input pin. If the required configuration of the port pin is an output pin, then the function Port_Init shall ensure that the default level is set before switching the functionality of the port pin from input to output.

[SWS_Port_00121] [The function Port_Init shall always have a pointer as a pa- rameter, even though for the configuration variant VARIANT-PRE-COMPILE, no con- figuration set shall be given. In this case, the Port Driver module’s environment shall pass a NULL pointer to the function Port_Init.♩(SRS_BSW_00414)

The Port Driver module’s environment shall not call the function Port_Init during a running operation. This shall only apply if there is more than one caller of the PORT module.

Configuration of Port_Init: All port pins and their functions, and alternate functions shall be configured by the configuration tool.

### Port_SetPinDirection [SWS_Port_00141] [

| Service Name | Port_SetPinDirection |  |
| --- | --- | --- |
| Syntax | void Port_SetPinDirection ( Port_PinType Pin, Port_PinDirectionType Direction ) |  |
| Service ID [hex] | 0x01 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | Pin | Port Pin ID number |
|  | Direction | Port Pin Direction |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Sets the port pin direction |  |
| Available via | Port.h |  |

♩()

[SWS_Port_00063] [The function Port_SetPinDirection shall set the port pin di- rection during runtime.♩(SRS_Port_12405)

[SWS_Port_00054] [The function Port_SetPinDirection shall be re-entrant if ac- cessing different pins independent of a port.♩()

[SWS_Port_00086] [The function Port_SetPinDirection shall only be available to the user if the pre-compile parameter PortSetPinDirectionApi is set to TRUE. If set to FALSE, the function Port_SetPinDirection is not available.♩(SRS_Port_- 12405)

Configuration of Port_SetPinDirection: All ports and port pins shall be configured by the configuration tool. See PORT117.

### Port_RefreshPortDirection

[SWS_Port_00142] [

| Service Name | Port_RefreshPortDirection |
| --- | --- |
| Syntax | void Port_RefreshPortDirection ( void ) |
| Service ID [hex] | 0x02 |
| Sync/Async | Synchronous |
| Reentrancy | Non Reentrant |
| Parameters (in) | None |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | None |
| Description | Refreshes port direction. |
| Available via | Port.h |

♩()

[SWS_Port_00060] [The function Port_RefreshPortDirection shall refresh the direction of all configured ports to the configured direction (PortPinDirection).♩ (SRS_Port_12406)

[SWS_Port_00061] [The function Port_RefreshPortDirection shall exclude those port pins from refreshing that are configured as ’pin direction changeable dur- ing runtime’.♩(SRS_Port_12406)

The configuration tool shall provide names for each configured port pin.

### Port_GetVersionInfo [SWS_Port_00143] [

| Service Name | Port_GetVersionInfo |  |
| --- | --- | --- |
| Syntax | void Port_GetVersionInfo ( Std_VersionInfoType* versioninfo ) |  |
| Service ID [hex] | 0x03 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | None |  |
| Parameters (inout) | None |  |
| Parameters (out) | versioninfo | Pointer to where to store the version information of this module. |
| Return value | None |  |
| Description | Returns the version information of this module. |  |
| Available via | Port.h |  |

♩()

[SWS_Port_00225] [if Det is enabled, the parameter versioninfo shall be

checked for being NULL. The error PORT_E_PARAM_POINTER shall be reported in case the value is a NULL pointer.♩()

### Port_SetPinMode [SWS_Port_00145] [

| Service Name | Port_SetPinMode |  |
| --- | --- | --- |
| Syntax | void Port_SetPinMode ( Port_PinType Pin, Port_PinModeType Mode ) |  |
| Service ID [hex] | 0x04 |  |
| Sync/Async | Synchronous |  |
| Reentrancy | Reentrant |  |
| Parameters (in) | Pin | Port Pin ID number |
|  | Mode | New Port Pin mode to be set on port pin. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | None |  |
| Description | Sets the port pin mode. |  |
| Available via | Port.h |  |

♩()

[SWS_Port_00125] [The function Port_SetPinMode shall set the port pin mode of the referenced pin during runtime.♩()

[SWS_Port_00128] [The function Port_SetPinMode shall be re-entrant if accessing different pins, independent of a port.♩()

[SWS_Port_00223] [If Det is enabled, the function Port_SetPinMode shall report

PORT_E_MODE_UNCHANGEABLE error and return without any other action, if the pa- rameter PortPinModeChangeable is set to FALSE.♩()

Configuration of Port_SetPinMode: All ports and port pins shall be configured by the configuration tool. See PORT117.

## Call-back notifications

There are no callback notifications from the PORT driver. The callback notifications are implemented in another module (ICU Driver and/or complex drivers).

## Scheduled functions

There are no scheduled functions within the PORT Driver.

## Expected Interfaces

In this chapter, all interfaces required from other modules are listed.

### Mandatory Interfaces

None

### Optional Interfaces

This chapter defines all interfaces which are required to fulfill an optional functionality of the module.

### [SWS_Port_00146] [

| API Function | Header File | Description |
| --- | --- | --- |
| Det_ReportError | Det.h | Service to report development errors. |

♩()

### Configurable Interfaces

None

# Sequence diagrams

User

Overall Configuration of Ports

«module» Port

Port_Init(const

Port_ConfigType*)

Port_Init()

Figure 9.1: Overall Configuration of Ports

User

Set the direction of a Port Pin

Port_SetPinDirection(Port_PinType, Port_

PinDirectionType)

Port_SetPinDirection()

«module» Port

Figure 9.2: Set the direction of a Port Pin

User

Refresh the direction of all Port Pins

Port_RefreshPortDirection()

Port_RefreshPortDirection()

«module» Port

Figure 9.3: Refresh the direction of all Port Pins

## Change the mode of a Port Pin

Figure 9.4: Change the mode of a Port Pin

# Configuration specification

In general, this chapter defines configuration parameters and their clustering into con- tainers. In order to support the specification Chapter 10.1 describes fundamentals. It also specifies a template (table) you shall use for the parameter specification. We intend to leave Chapter 10.1 in the specification to guarantee comprehension.

Section 10.2 specifies the structure (containers) and the parameters of the module PORT

Section 10.3 specifies published information of the module PORT.

## How to read this chapter

For details refer to the Chapter 10.1 "Introduction to configuration specification" in [4].

## Containers and configuration parameters

The following chapters summarize all configuration parameters. The detailed meanings of the parameters describe Chapters 7 and Chapter 8.

[SWS_Port_00232] [The PORT module shall reject configurations with partition map- pings which are not supported by the implementation.♩()

### Port

| SWS Item | [ECUC_Port_00135] |
| --- | --- |
| Module Name | Port |
| Description | Configuration of the Port module. |
| Post-Build Variant Support | true |
| Supported Config Variants | VARIANT-POST-BUILD, VARIANT-PRE-COMPILE |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| PortConfigSet | 1 | This container contains the configuration parameters and sub containers of the AUTOSAR Port module. |
| PortGeneral | 1 | Module wide configuration parameters of the PORT driver. |

+container

+container

PortConfigSet: EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 1

upperMultiplicity = 1 lowerMultiplicity = 0

PortGeneral:  EcucParamConfContainerDef

Port: EcucModuleDef

Figure 10.1: Port Configuration Overview

### PortContainer

| SWS Item | [ECUC_Port_00122] |
| --- | --- |
| Container Name | PortContainer |
| Parent Container | PortConfigSet |
| Description | Container collecting the PortPins. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Port_00124] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortNumberOfPortPins |  |  |
| Parent Container | PortContainer |  |  |
| Description | The number of specified PortPins in this PortContainer. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef |  |  |
| Range | 1 .. 65535 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| Included Containers |  |  |
| --- | --- | --- |
| Container Name | Multiplicity | Scope / Dependency |
| PortPin | 1..* | Configuration of the individual port pins. |

+literal

+literal

+container

+literal

+literal

+subContainer

+literal

+literal

+literal

+subContainer

+parameter

+literal

+literal

+literal

+literal

PortPin:  EcucParamConfContainerDef

upperMultiplicity = * lowerMultiplicity = 1

upperMultiplicity = * lowerMultiplicity = 1

PortContainer:  EcucParamConfContainerDef

PortConfigSet: EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

Port: EcucModuleDef

PORT_PIN_MODE_FLEXRAY:

EcucEnumerationLiteralDef

PORT_PIN_MODE_LIN:

EcucEnumerationLiteralDef

PORT_PIN_MODE_CAN:

EcucEnumerationLiteralDef

PORT_PIN_MODE_ICU:

EcucEnumerationLiteralDef

PORT_PIN_MODE_SPI:

EcucEnumerationLiteralDef

PORT_PIN_MODE_PWM:

EcucEnumerationLiteralDef

PORT_PIN_MODE_ADC:

EcucEnumerationLiteralDef

PORT_PIN_MODE_MEM:

EcucEnumerationLiteralDef

PORT_PIN_MODE_DIO_WDG:

EcucEnumerationLiteralDef

PORT_PIN_MODE_DIO_GPT:

EcucEnumerationLiteralDef

lowerMultiplicity = 1 upperMultiplicity = *

PORT_PIN_MODE_DIO:

EcucEnumerationLiteralDef

PortPinMode:  EcucEnumerationParamDef

Figure 10.2: PortContainer Parameters Overview

### PortGeneral

| SWS Item | [ECUC_Port_00117] |
| --- | --- |
| Container Name | PortGeneral |
| Parent Container | Port |
| Description | Module wide configuration parameters of the PORT driver. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Port_00123] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortDevErrorDetect |  |  |
| Parent Container | PortGeneral |  |  |
| Description | Switches the development error detection and notification on or off. - true: detection and notification is enabled. - false: detection and notification is disabled. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |

q

Δ

|  | Link time | – |  |
| --- | --- | --- | --- |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00131] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortSetPinDirectionApi |  |  |
| Parent Container | PortGeneral |  |  |
| Description | Pre-processor switch to enable / disable the use of the function Port_SetPinDirection(). TRUE: Enabled - Function Port_SetPinDirection() is available. FALSE: Disabled - Function Port_SetPinDirection() is not available. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00132] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortSetPinModeApi |  |  |
| Parent Container | PortGeneral |  |  |
| Description | Pre-processor switch to enable / disable the use of the function Port_SetPinMode(). true: Enabled - Function Port_SetPinMode() is available. false: Disabled - Function Port_SetPinMode() is not available. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00133] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortVersionInfoApi |  |  |
| Parent Container | PortGeneral |  |  |
| Description | Pre-processor switch to enable / disable the API to read out the modules version information. true: Version info API enabled. false: Version info API disabled. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | false |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00136] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortEcucPartitionRef |  |  |
| Parent Container | PortGeneral |  |  |
| Description | Maps the Port driver to zero a multiple ECUC partitions to make the modules API available in this partition. |  |  |
| Multiplicity | 0..* |  |  |
| Type | Reference to EcucPartition |  |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

The top level Port Driver container holds parameters that apply to the PORT configu- ration.

+parameter

+parameter

+container

+parameter

+parameter

defaultValue = false

PortVersionInfoApi: EcucBooleanParamDef

PortSetPinModeApi:  EcucBooleanParamDef

upperMultiplicity = 1 lowerMultiplicity = 0

Port: EcucModuleDef

PortSetPinDirectionApi:  EcucBooleanParamDef

upperMultiplicity = 1 lowerMultiplicity = 1

defaultValue = false

PortDevErrorDetect: EcucBooleanParamDef

PortGeneral:  EcucParamConfContainerDef

|  |  | EcucPartition: EcucParamConfContainerDef |
| --- | --- | --- |
| PortEcucPartitionRef: EcucReferenceDef | +destination |  |
|  |  | lowerMultiplicity = 0 upperMultiplicity = * |
| lowerMultiplicity = 0 upperMultiplicity = * |  |  |

+reference

Figure 10.3: PortGeneral Parameters Overview

### PortPin

| SWS Item | [ECUC_Port_00118] |
| --- | --- |
| Container Name | PortPin |
| Parent Container | PortContainer |
| Description | Configuration of the individual port pins. |
| Configuration Parameters |  |

| SWS Item | [ECUC_Port_00125] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinDirection |  |  |
| Parent Container | PortPin |  |  |
| Description | The initial direction of the pin (IN or OUT). If the direction is not changeable, the value configured here is fixed. The direction must match the pin mode. E.g. a pin used for an ADC must be configured to be an in port. Implementation Type: Port_PinDirectionType |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range | PORT_PIN_IN | Port Pin direction set as input |  |
|  | PORT_PIN_OUT | Port Pin direction set as output |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00126] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinDirectionChangeable |  |  |
| Parent Container | PortPin |  |  |
| Description | Parameter to indicate if the direction is changeable on a port pin during runtime. true: Port Pin direction changeable enabled. false: Port Pin direction changeable disabled. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00127] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinId |  |  |
| Parent Container | PortPin |  |  |
| Description | Pin Id of the port pin. This value will be assigned to the symbolic name derived from the port pin container short name. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucIntegerParamDef (Symbolic Name generated for this parameter) |  |  |
| Range | 1 .. 65535 |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | false |  |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00128] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinInitialMode |  |  |
| Parent Container | PortPin |  |  |
| Description | Port pin mode from mode list for use with Port_Init() function. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range | PORT_PIN_MODE_ADC | Port Pin used by ADC |  |
|  | PORT_PIN_MODE_CAN | Port Pin used for CAN |  |
|  | PORT_PIN_MODE_DIO | Port Pin configured for DIO. It shall be used under control of the DIO driver. |  |
|  | PORT_PIN_MODE_DIO_GPT | Port Pin configured for DIO. It shall be used under control of the general purpose timer driver. |  |
|  | PORT_PIN_MODE_DIO_WDG | Port Pin configured for DIO. It shall be used under control of the watchdog driver. |  |
|  | PORT_PIN_MODE_FLEXRAY | Port Pin used for FlexRay |  |
|  | PORT_PIN_MODE_ICU | Port Pin used by ICU |  |
|  | PORT_PIN_MODE_LIN | Port Pin used for LIN |  |
|  | PORT_PIN_MODE_MEM | Port Pin used for external memory under control of a memory driver. |  |
|  | PORT_PIN_MODE_PWM | Port Pin used by PWM |  |
|  | PORT_PIN_MODE_SPI | Port Pin used by SPI |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00129] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinLevelValue |  |  |
| Parent Container | PortPin |  |  |
| Description | Port Pin Level value from Port pin list. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucEnumerationParamDef |  |  |
| Range | PORT_PIN_LEVEL_HIGH | Port Pin level is High |  |
|  | PORT_PIN_LEVEL_LOW | Port Pin level is LOW |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00130] |
| --- | --- |
| Parameter Name | PortPinMode |
| Parent Container | PortPin |
| Description | Port pin mode from mode list. Note that more than one mode is allowed by default. That way it is e.g. possible to combine DIO with another mode such as ICU. |
| Multiplicity | 1..* |
| Type | EcucEnumerationParamDef |

q

Δ

| Range | PORT_PIN_MODE_ADC | Port Pin used by ADC |  |
| --- | --- | --- | --- |
|  | PORT_PIN_MODE_CAN | Port Pin used for CAN |  |
|  | PORT_PIN_MODE_DIO | Port Pin configured for DIO. It shall be used under control of the DIO driver. |  |
|  | PORT_PIN_MODE_DIO_GPT | Port Pin configured for DIO. It shall be used under control of the general purpose timer driver. |  |
|  | PORT_PIN_MODE_DIO_WDG | Port Pin configured for DIO. It shall be used under control of the watchdog driver. |  |
|  | PORT_PIN_MODE_FLEXRAY | Port Pin used for FlexRay |  |
|  | PORT_PIN_MODE_ICU | Port Pin used by ICU |  |
|  | PORT_PIN_MODE_LIN | Port Pin used for LIN |  |
|  | PORT_PIN_MODE_MEM | Port Pin used for external memory under control of a memory driver. |  |
|  | PORT_PIN_MODE_PWM | Port Pin used by PWM |  |
|  | PORT_PIN_MODE_SPI | Port Pin used by SPI |  |
| Post-Build Variant Multiplicity | true |  |  |
| Post-Build Variant Value | true |  |  |
| Multiplicity Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00134] |  |  |
| --- | --- | --- | --- |
| Parameter Name | PortPinModeChangeable |  |  |
| Parent Container | PortPin |  |  |
| Description | Parameter to indicate if the mode is changeable on a port pin during runtime. True: Port Pin mode changeable allowed. False: Port Pin mode changeable not permitted. |  |  |
| Multiplicity | 1 |  |  |
| Type | EcucBooleanParamDef |  |  |
| Default value | – |  |  |
| Post-Build Variant Value | true |  |  |
| Value Configuration Class | Pre-compile time | X | VARIANT-PRE-COMPILE |
|  | Link time | – |  |
|  | Post-build time | X | VARIANT-POST-BUILD |
| Scope / Dependency | scope: local |  |  |

| SWS Item | [ECUC_Port_00137] |
| --- | --- |
| Parameter Name | PortPinEcucPartitionRef |
| Parent Container | PortPin |
| Description | Maps the Port pin to zero a multiple ECUC partitions. The ECUC partitions referenced are a subset of the ECUC partitions where the Port driver is mapped to. |
| Multiplicity | 0..* |
| Type | Reference to EcucPartition |
| Post-Build Variant Multiplicity | true |
| Post-Build Variant Value | true |

q

Δ

| Multiplicity Configuration Class | Pre-compile time | X | All Variants |
| --- | --- | --- | --- |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Value Configuration Class | Pre-compile time | X | All Variants |
|  | Link time | – |  |
|  | Post-build time | – |  |
| Scope / Dependency | scope: ECU |  |  |

No Included Containers

Port: EcucModuleDef

+container

PortConfigSet: EcucParamConfContainerDef

upperMultiplicity = 1 lowerMultiplicity = 0

+subContainer

PortContainer:  EcucParamConfContainerDef

PortNumberOfPortPins:

+parameter	EcucIntegerParamDef

upperMultiplicity = *	min = 1

lowerMultiplicity = 1	max = 65535

+subContainer

PortPin:  EcucParamConfContainerDef

PortPinId: EcucIntegerParamDef

+parameter

upperMultiplicity = * lowerMultiplicity = 1

min = 1

symbolicNameValue = true max = 65535

+parameter PortPinDirectionChangeable:

EcucBooleanParamDef

PortPinLevelValue:

+parameter  EcucEnumerationParamDef

+literal PORT_PIN_LEVEL_HIGH:  EcucEnumerationLiteralDef

+literal

PORT_PIN_LEVEL_LOW:

EcucEnumerationLiteralDef

+literal

PortPinDirection:

+parameter  EcucEnumerationParamDef

PORT_PIN_OUT:

EcucEnumerationLiteralDef

+literal

PORT_PIN_IN:

EcucEnumerationLiteralDef

PortPinMode:

+parameter  EcucEnumerationParamDef

lowerMultiplicity = 1 upperMultiplicity = *

+parameter	PortPinInitialMode:

EcucEnumerationParamDef

+parameter  PortPinModeChangeable:

EcucBooleanParamDef

+reference

PortPinEcucPartitionRef:  EcucReferenceDef

EcucPartition:

lowerMultiplicity = 0 upperMultiplicity = *

+destination  EcucParamConfContainerDef

lowerMultiplicity = 0 upperMultiplicity = *

Figure 10.4: PortPin Parameters Overview

## Constraints

[SWS_Port_CONSTR_00233] [The ECUC partitions referenced by PortPinEcuc- PartitionRef shall be a subset of the ECUC partitions referenced by PortEcuc- PartitionRef.♩()

[SWS_Port_CONSTR_00234] [If PortEcucPartitionRef references one or more ECUC partitions, PortPinEcucPartitionRef shall have a multiplicity of greater than zero and reference one or several of these ECUC partitions as well.♩()

## Published Information

For details refer to the Chapter 10.3 "Published Information" in [4].

# A	Not applicable requirements

[SWS_Port_NA_00227] [These requirements are not applicable to this specifica- tion.♩(SRS_BSW_00005, SRS_BSW_00006, SRS_BSW_00007, SRS_BSW_00010, SRS_BSW_00160, SRS_BSW_00161, SRS_BSW_00162, SRS_BSW_00164, SRS_-

| BSW_00167, | SRS_BSW_00168, | SRS_BSW_00170, | SRS_BSW_00172, | SRS_- |
| --- | --- | --- | --- | --- |
| BSW_00307, | SRS_BSW_00308, | SRS_BSW_00309, | SRS_BSW_00321, | SRS_- |
| BSW_00325, | SRS_BSW_00328, | SRS_BSW_00330, | SRS_BSW_00331, | SRS_- |
| BSW_00333, | SRS_BSW_00334, | SRS_BSW_00335, | SRS_BSW_00336, | SRS_- |
| BSW_00341, | SRS_BSW_00342, | SRS_BSW_00343, | SRS_BSW_00344, | SRS_- |
| BSW_00347, | SRS_BSW_00357, | SRS_BSW_00359, | SRS_BSW_00360, | SRS_- |
| SPAL_12463, | SRS_SPAL_12462, | SRS_SPAL_12265, | SRS_SPAL_12092, | SRS_- |
| SPAL_12078, | SRS_SPAL_12077, | SRS_SPAL_12067, | SRS_SPAL_12064, | SRS_- |
| SPAL_12129, | SRS_SPAL_12075, | SRS_SPAL_12063, | SRS_SPAL_12169, | SRS_- |
| SPAL_00157, | SRS_SPAL_12069, | SRS_SPAL_12068, | SRS_SPAL_12267, | SRS_- |
| SPAL_12056, | SRS_BSW_00440, | SRS_BSW_00439, | SRS_BSW_00437, | SRS_- |
| BSW_00433, | SRS_BSW_00432, | SRS_BSW_00429, | SRS_BSW_00428, | SRS_- |
| BSW_00427, | SRS_BSW_00426, | SRS_BSW_00425, | SRS_BSW_00424, | SRS_- |
| BSW_00423, | SRS_BSW_00419, | SRS_BSW_00417, | SRS_BSW_00416, | SRS_- |
| BSW_00413, | SRS_BSW_00398, | SRS_BSW_00395, | SRS_BSW_00378, | SRS_- |

BSW_00377, SRS_BSW_00375, SRS_BSW_00373, SRS_BSW_00371)